TINY BASIC SUMMARY

Editing		Standard Basic

Direct Mode	All Verbs Usable

Statement Types

PRINT	Item List

LET	Var = Expr	(LET is optional)

IF	Expr Relop Expr  Statement

INPUT	Variable List

GOTO	Line Number

GOSUB	Line Number

RETURN

POKE	POKE(Expr) = Expr

STOP

LIST	Line Number, Line Number	(Line Numbers are optional)

RUN

NEW

Functions

USR	Variable = USR(Expr,Expr)

PEEK	Variable = PEEK(Expr)

MEM	Variable = MEM

Number		Integers to _#+32767 or Hex Integers preceded by a $ symbol

Variable	Letters A-Z

Expression	Variables, Numbers, and Functions combined with the following

operators +, -, *, /, (, ).

Relop		Comparison operators =, <, >, <=, >=, <>.

Line Number	Numbers 1 through 9999

String		"ALPHANUMERICS"

Item List	Expressions and Strings seperated by format control

characters , and ;.

Control Chars.	Control H or "Back Space" deletes last input character.

Control X or "Cancel" deletes entire input line.

Control C Terminates Basic program or List operation and

returns control to command mode.

Memory Usage	Tiny Basic V1.37

$0080 - $009F	Tiny Basic interpreter scratch area.

$00A0 - $00FD	Not used by Tiny Basic interpreter. (usable USR routines)

$**** - $****	Pointer to Interrupt Vector Table. (Identical to LILBUG)

$D800 - $DFFF	Input Buffer, Basic Program storage, Stack Space, and

Variables in RAM.

$**** - $****	Optional Power Up Basic Program and/or USR functions in ROM.

$E800 - $EFFF	Tiny Basic interpreter ROM.

$E800		Cold Start Address.

$E803		Warm Start Address.

Tiny Basic USR Function

The USR function in Tiny Basic takes 2 arguments and returns a value to a

variable. The form of the USR function is "LET V = USR(Expr,Expr)".

The USR function can be used in any expression in Tiny Basic as an example

"LET V = A * ( B + USR( $EF00, K))". The USR function can also be used with

the PRINT statement.

The first argument of the USR function is evaluated to determine the address

or the machine language code to be called. The second argument is evaluated

and the value is send to the machine code routine in the D accumulator. The

second argument is optional, if it is present the Carry bit in the condition

code register will be cleared when the machine code routine is called. If the

second argument is not present the Carry Bit will be set when the machine code

is called. The machine code routine may return a result to the BASIC program

in the D accumulator, the value in the D accumulator on return from the machine

code routine will be used by the BASIC program as the value of the function.

The machine code routine must execute a RTS instruction to return conterol to

the BASIC program. The machine code routine may use all the processor registers

freely and need not save and restore any registers. It is important that the

machine code routine not modify any memory used by the Tiny Basic interpreter.

Consult the memory map provided with your version of Tiny Basic to determine

which memory areas are not used.

Tiny Basic handles interrupts with the same interrupt vectoring technique used

by LILBUG. Consult the LILBUG manual for details on interrupt vector usage.

JPB  12-APR-82