USER PROGRAM, GENERAL TECHNICAL DOCUMENTATION

The sim6809 program can load an user program for simulation of

hardware. The user program can be in three different formats,

BIN, COM or EXE. It may be written in any languages as long as

it obeys the following rules:

# The startaddress must be at offset 0 for .BIN files, at offset

100H for .COM files and at the specified value in the EXE-Header

(same as link.exe reports) for .EXE files.

# A near jumptable to far procedures must start the program:

startaddress+0:       jmp no_exec_prog

startaddress+3:       jmp clock_update

startaddress+6:       jmp startup

startaddress+9:       jmp terminate

startaddress+12:      jmp reset

startaddress+15:      jmp read_io

startaddress+18:      jmp write_io

startaddress+21:      jmp dump_io

startaddress+24:      jmp enter_io

startaddress+27:      jmp watch_io

startaddress+30:      jmp sync_instruction

See below for arguments and return values for each procedure.

# On all procedure entries in the jumptable the DI, SI, BP, DS,

SS and SP registers must be saved. On return, and on calls to the

sim6809's user function except for DI, SI, BP and SP, these are

restored and the Direction Flag must be cleared and the Interrupt

Flag set.

# The DS register has the same value as the SS register and

need not to be saved if it is restored with SS.

# The user program need not to setup it's own stack (and may

therefore not save the SS:SP registers). The sim6809.exe

has 2Kb extra stackspace for the user program.

# The user program must NOT call such MS-DOS or BIOS functions that

writes to the screen or reads from the keyboard or mouse, only

calls to the sim6809's user function are allowed.

# The user program must NOT change any interrupt vectors.

(Note, highlevel-languages may use vectors for different tasks,

e.i. floating point).

# The sim6809.exe program has a floating point emulation that

uses the coprocessor if it is installed. The emulation is done

by Microsoft C version 5.00.

USER PROGRAM

# Note, the user program has no PSP or Environment. It is loaded as

an overlay (MS-DOS function 4BH code 03H) and is a part of the

sim6809.exe program.

# Arguments are passed to the user program on the stack (SS:SP).

# Arguments are passed from the user program on the same stack

(SS restored) via a pusha instruction.

# Return values to the user program are placed on the same stack

and after a popa instruction they are placed in registers.

# Return values from the user program are in the AX and DX register.

SUPPORTED PROCEDURES BY THE USER PROGRAM

Each procedure is of far type and most end with a far ret

instruction. Arguments on the stack shall not be removed,

it is done by the calling procedure (sim6809.exe).

The procedures are called by sim6809.exe through the jumptable

at the user program's startaddress.

Use the following algorithm in your procedures:

proc_name       PROC FAR

push bp

mov bp,sp

.

.                     ;Save other registers

.

mov ax,[bp+6]           ;Argument 1

.

mov ax,[bp+8]           ;Argument 2

.

mov ax,[bp+4+2*n]       ;Argument n

.

.

mov ax,return_value1

mov dx,return_value2

.

.                     ;Restore registers

.

mov sp,bp               ;If needed

pop bp

ret                     ;Return value in DX:AX

proc_name       ENDP

See the exemain.asm source file for one example of how to implement

a general user program interface for assembly programs.

See the msc500.asm source file for one example of how to implement

a general user program interface for Microsoft C version 5.00.

In the explanation below 'arg1' to 'argn' are the arguments from

sim6809.exe placed on the stack. AX and DX are the return values

from the user program procedure.

SUPPORTED PROCEDURES BY THE USER PROGRAM

startaddress+0:       jmp no_exec_prog

This procedure is never called by the sim6809.exe program.

It is used for COM and EXE user programs if the program is

loaded by MS-DOS. Normal action is to write a message as

"No executable program" and terminate.

startaddress+3:         jmp clock_update

IN:     arg1:   Number of clockcycles since the last call

OUT:    AX:     Signal status, bit 0 = RESET

bit 1 = IRQ

bit 2 = NMI

bit 3 = FIRQ

bit 4-7 must be set to zero

bit 8 = userprogram event

DX:     Don't care

This procedure is called after each simulated instruction and

should be implemented as fast as possible. The returned signal

is immediately simulated.

startaddress+6:         jmp startup

IN:     arg1:   Segment value for the allocated memory in use by the

userprogram.

arg2:   Segment size in paragraphs

arg3:   EXE-Header SS value, for EXE files only

arg4:   EXE-Header SP value, for EXE files only

arg5:   Offset address to the sim6809 user function entry.

arg6:   Segment address to the sim6809 user function entry.

OUT:    AX:     Version in hexadecimal (vers 1.01 = 0101H),

or FFFFH if not enough memory for the userprogram.

DX:     Processor (CPU) type. For the sim6809.exe the CPU

type is 0.

This procedure is called only once and it should save the arguments

for future use. The MS-DOS function 4AH (Set block) must be

called (with arg1 in ES) to free unused memory.

SUPPORTED PROCEDURES BY THE USER PROGRAM

startaddress+9:         jmp terminate

IN:     No arguments

OUT:    AX:     Don't care

DX:     Don't care

When this procedure is called it must free any additional allocated

memory (outside the program memory block), close all files etc, and

then return (not call any MS-DOS terminate functions). Note, the

program memory block should not be freed. Note, this procedure is

also called when the startup procedure returns FFFFH.

startaddress+12:        jmp reset

IN:     No arguments

OUT:    AX:     Don't care

DX:     Don't care

This procedure is called when the RESET signal is active.

startaddress+15:        jmp read_io

IN:     arg1:   I/O address

arg2:   I/O base address (defined in *.sim)

OUT:    AX:     Read value (8 bit)

DX:     0 --> AX is valid

1 --> Unknown (not supported) I/O address

This procedure is called when a read of an I/O is done.

startaddress+18:        jmp write_io

IN:     arg1:   I/O address

arg2:   I/O base address (defined in *.sim)

arg3:   Write value (8 bit)

OUT:    AX:     Don't care

DX:     0 --> Value is written

1 --> Unknown (not supported) I/O address

This procedure is called when a write to an I/O is done.

SUPPORTED PROCEDURES BY THE USER PROGRAM

startaddress+21:        jmp dump_io

IN:     arg1:   I/O base address

OUT:    AX:     Don't care

DX:     0 --> I/O is dumped

1 --> Unknown (not supported) I/O base address

This procedure is called when a dump of an I/O is wanted.

It should open a window and display the wanted I/O.

startaddress+24:        jmp enter_io

IN:     arg1:   I/O base address

OUT:    AX:     Don't care

DX:     0 --> I/O is entered

1 --> Unknown (not supported) I/O base address

This procedure is called when a enter of an I/O is wanted.

It should open a window and ask for values.

startaddress+27:        jmp watch_io

IN:     arg1:   I/O base address

OUT:    AX:     Don't care

DX:     0 --> I/O is dumped to the window

1 --> Unknown (not supported) I/O base address

This procedure is called when a watch of an I/O is wanted and when

the watch-window is updated. The user programs window is set to

the line in the watch-window where the watch-data should be placed

by this user function.

startaddress+30:        jmp sync_instruction

IN:     No arguments

OUT:    AX:     0 --> Continue with the sync instruction

1 --> End the sync instruction

DX:     Don't care

This procedure is called when the CPU is in the SYNC state. The

simulated program is continued only when the return value is

nonzero (simulates a short IRQ or FIRQ). Note that the 'clock_update'

procedure is called during the sync-wait and that it may end the

sync instruction.

SUPPORTED FUNCTIONS BY THE SIM6809 USER FUNCTION

To call the user function entry point use the following

algorithm, so all arguments and return values are

passed in registers:

# Save user SS:SP (own stack)

# Restore sim6809 SS:SP (own stack)

# Execute a pusha 80286 instruction

# Push es

# Push ds

# Load DS with SS (if changed)

# Clear the direction flag (if changed)

# Set the interrupt flag (if changed)

# Do a far call to the user function entry point

# Pop ds

# Pop es

# Execute a popa 80286 instruction

# Restore user SS:SP (own stack)

FUNCTIONS:

inkey           in:     AH = 00H

out:    AX = ASCII code (Extended)

Gets a key from the keyboard.

openwind        in:     AH = 01H

DS:DX = ASCIIZ string

BL = Column size

BH = Row size

out:    none

Opens a window.

SUPPORTED FUNCTIONS BY THE SIM6809 USER FUNCTION

setcur          in:     AH = 02H

BL = Column in window

BH = Row in window

out:    none

Sets cursor position in the window.

getcur          in:     AH = 03H

out:    BL = Column in window

BH = Row in window

Gets cursor position in the window.

printspace      in:     AH = 04H

BX = Number of spaces to print

out:    none

Prints spaces in window, useful in advancing the cursor position.

printstr        in:     AH = 05H

DS:DX = ASCIIZ string

out:    none

Prints string in window.

printhex        in:     AH = 06H

AL = Number of digits (1-8)

DX:BX = value

out:    none

Prints hexadecimal value in window.

printbin        in:     AH = 07H

AL = Number of digits (1-32)

DX:BX = value

out:    none

Prints binary value in window.

SUPPORTED FUNCTIONS BY THE SIM6809 USER FUNCTION

printdec        in:     AH = 08H

AL = Number of digits (1-10)

DX:BX = value

out:    none

Prints signed decimal value in window.

inputstr        in:     AH = 09H

DS:DX = Default ASCIIZ string

out:    AX = ENTER or ESC key

Inputs ASCIIZ string from keyboard to DS:DX.

inputhex        in:     AH = 0AH

AL = Number of digits (1-8)

DX:BX = Default value

out:    AX = ENTER or ESC key

DX:BX = Inputed value (if AX = ENTER)

Inputs hexadecimal value from keyboard.

inputbin        in:     AH = 0BH

AL = Number of digits (1-32)

DX:BX = Default value

out:    AX = ENTER or ESC key

DX:BX = Inputed value (if AX = ENTER)

Inputs binary value from keyboard.

inputdec        in:     AH = 0CH

AL = Number of digits (1-10)

DX:BX = Default value

out:    AX = ENTER or ESC key

DX:BX = Inputed value (if AX = ENTER)

Inputs signed decimal value from keyboard.

newline         in:     AH = 0DH

out:    none

If the cursor isn't in the first column a "newline" is done.

SUPPORTED FUNCTIONS BY THE SIM6809 USER FUNCTION

scroll          in:     AH = 0EH

AL = number of lines to scroll (up or down)

out:    none

Scrolls the window up or down (positive value means ordinary scroll).

closewind       in:     AH = 0FH

out:    none

Closes the window.

printerr        in:     AH = 10H

DS:DX = ASCIIZ string

out:    none

Prints an error message.