$a pageno=0

$b18

$l0m

A Novel

Digital to Analogue

Converter

$b30

$c40 L.D.$ Smith.   $c70 June 1983.

$n

$A CAP=0; CAPSH=0; UND='_'; UNDSH='%'; INVERT=0

$A TAB=4,7,10,LINE

$l1m

Contents

$b4

0.$ $ Background

$T4  $C-1 1 $B0

$...$ The Problem  $t4 $c-1 1 $b0

$b2

1.$ $ A$ First$ Hack

$T4  $C-1 2 $B0

$b

1.1$ The$ Chip$ returns...

$T4  $C-1 3 $B0

$...$ Testing and Performance $t4 $c-1 3 $b0

$b2

2.$ $ Version$ Two...

$T4  $C-1 4 $B0

$b2

3.$ $ Version Three...

$T4  $c-1 4 $B0

$b2

$ $ $ $ Appendix 1

$T4  $c-3 1-1 $b0

$b2

$ $ $ $ Appendix 2

$t4  $c-3 2-1 $b0

$...$ Binary to Unary Pipeline Tree Floorplan $b0

$...$ Pipelined Binary to Unary Tree Stage $b0

$...$ Tree Cell Layout $b0

$b2

$ $ $ $ Appendix 3 $t4 $c-3 3-1 $b0

$b

Calculating the Sizes of the Switched Current Sources $t4 $c-3 3-1 $b0

SPICE Input Produced by Program sjsrc $t4 $c-3 3-2 $b0

An extract from the SPICE Output $t4 $c-3 3-3 $b0

Least Squares Fit and Deviation from It $t4 $c-3 3-3 $b0

$...$ Plot of the Analogue Output $b0

$b

Layout of the DtoA Converter $t4 $c-3 3-4 $b0

$...$ Binary (2) -> Unary (2**2) Converter Ploorplan $b0

$...$ Transistor Diagram of the Switched Current Source Driver Cct. $b0

$...$ SPICE Predicted Performance of the Driver Circuit $b0

$...$ Layout of a Pair of Driver Circuits $b0

$...$ Transistor Diagram of the Bit-Line Driver Cct. $b0

$...$ SPICE Predicted Performance of the Bit-Line Driver with TTL Input $b0

$...$ Layout of the Bit-Line Driver $b0

$...$ Layout of an Analogue Output Cell Pair $b0

$...$ Pad Plan of the Complete Chip $b0

$...$ Layout of the Complete Chip $b0

$...$ Proposed Test Rig $b0

$...$ Timing Waveforms of the Proposed Test Rig $b0

$E

$a top=2;bottom=2

$a page=40;dlpi=4;sup='^';sub='_'

$a pgap=0

$l1um

A Novel Digital To Analogue Converter

$b2

$l1uh

0.  Background

$p

In the Department of Computer Science at Edinburgh University there are

three raster-scan colour-graphics terminals used for integrated circuit

design which have been built locally, based on a design by Charles Minter

at the California Institute of Technology. Although each terminal is

different (!), the essence of their operation is that a stream of 8-bit

pixels is clocked out of the frame-store at a peak rate of 12.5MHz (84ns

per pixel). Each byte is reduced to 6-bits by random logic and then the

6-bit reduced pixel value indexes into a 64 X 12-bit fast (35ns) RAM

which implements a 'colour map'. The 12-bit output is interpreted as

three 4-bit Red, Blue and Green values which are converted linearly to

analogue signals in the range 0-1V prior to driving a colour monitor

down three 75$ Ohm coaxial cables.

At the present time, the digital to analogue (DtoA) conversion is performed

by a switched resistor ladder using open collector TTL circuits as switches.

The colour monitors used feature automatic dark-level adaption,

manual contrast and brightness controls,

but need to be pre-set to the characteristics of each individual DtoA converter

if the %range of output of each is not the same. This is a complex and

time-consuming task.

$p

Several points should be noted. Firstly, the %range of each of the three

DtoA converters used must be identical (as noted above) if re-tuning the

factory pre-set monitors is to be avoided.  Secondly, there are no real

linearity constraints involved:  a pixel picks one of 4095 possible 'shades'

from a palette of 64 which are concurrently displayable. The translation

is via a table (the colour map) of magic numbers so non-linearity is

unimportant: it merely alters the numbers which have to be loaded (and,

of course, slightly alters the choice of available shades).  Thirdly,

there are strict speed and impedance constraints to meet: the DtoA converter

%must drive 75$ Ohm coaxial cable at 12.5MHz. Fourthly, the current DtoA

converters using resistor ladders and TTL gates are messy, unmatched

(demanding monitor tuning) and consume excessive board area and wiring space.

$p

On the basis of these observations it was concluded that it would be a

potentially useful exercise to attempt to integrate three DtoA converters,

the colour map, bit compression circuitry and blanking circuitry on to a

single chip.

It was recognised, at this stage, that the attempt was unlikely to meet

its speed constraints using the 6u EMF single poly Si-gate nMOS fabrication

process. However, the ease (triviality) of testing the design made it

attractive to a busy academic with limited access to sophisticated test

equipment.

$b2$v5

$l1uh

1.  A First Hack

$p

In December 1981, towards the end of 1st VLSI Design course, it was decided

to proceed with a design for the DtoA part of the proposed device. The

design submission deadline was only about 10 days off at that time and there

were problems with the CAD software (ILAP) caused by panic modifications

designed to eliminate butting contacts, which had yielded very badly, from

the programmed layout generators. A simple design, requiring the use of no

programmed layout generators seemed an ideal choice.  Back of the envelope

calculations revealed

that the design was on using a sequence of (approximate) switched current

sources of size 2^0, 2^1, 2^2, 2^3 and 2^4 (a 5-bit DtoA was submitted).

A depletion mode MOSFET in series with an enhancement mode MOSFET of the same

size were used as an approximation to a switched current source. The drain

of the depletion transistor was fed from Vdd (5V) and the source of the

enhancement transistor was connected to the analogue output.  Channel

widths were calculated to drive 1V into 75$ Ohms with all sources turned

on. The total channel width was 651u and the calculated linearity over

0-1V was fairly good, being within 10$% of a least squares linear approximation

to the output over the full range.

In addition, it was necessary to design a gate-driver circuit to restore

TTL input signals to MOS levels and to drive the load imposed by the largest

gate in the current source (W=336u, L=6U).  A high ratio inverter followed

by an inverting super-buffer was used, all designed using Mead and Conway

approximate electrical rules.

$p

System design took about 1 day, including the time taken to write a Pascal

program to solve the Schichman-Hodges equations for the MOSFETs in the

current source and tabulate the output for inputs 0..31.  Layout took

a further day and involved the design of two cells, one of which (the

current source) could be programmed to be generated with any width.

Placement and wiring of pads took a further half day.

$p

Two copies of the design were submitted within the starting frame, mirrored

about the Y-axis (there were two copies of the DtoA, in the same orientation,

in each die). The design was submitted for fabrication and returned for

testing at the beginning of April$ 1982.

$b$v5

$l1uh

1.1 The Chip returns...

$p

The design was successfully fabricated and returned for functional testing.

The DtoA converters were driven from TTL synchronous counters (74S163, 4-bit

+ve edge triggered binary counters) and monitored on a 50MHz Texas oscilloscope.

There were several problems.

$p

Firstly, no allowance had been made for the shortening or narrowing of

drawn channels in the EMF process.  This caused the output to swing between

0V and 1.6V with a commensurate loss of linearity (the output is approximately

parabolic).

$p

Secondly, a digital 'glitch'

when the input value changes between 2^n and 2^n- 1 was overlooked. This

problem became more pronounced as the clock speed was turned up.

$p

Thirdly, the design failed to operate at the target speed. Operation at 1MHz

was very satisfactory, but the output became lumpy and glitch riddled at less

than 5MHz.

$p

On the positive side, both DtoA converters on the same chip appeared to

be perfectly matched, as did converters on different chips from the same

wafer.

$p

The poor performance of the design was investigated further. Firstly, a

complete SPICE2G.5 simulation of the circuit (40 MOSFETs) operating

at 12.5MHz was performed. This revealed the digital glitch problem very

clearly and highlighted a lack of drive capability in the gate-driver

driving the most significant current source.  This still left a factor

of 2.5 to be explained away: the SPICE simulation at 12.5MHz showed the

same characteristics as the oscilloscope did at 5MHz. Consequently, a

pencil and paper stability analysis of the current source was undertaken, but

with negative conclusions: no oscillatory tendancies could be found. Indeed,

the damping appeared to be very good, with very short settling times as

shown by the SPICE simulation. It was finally concluded that the poor

performance could only be attributed to fabrication factors such as

nitride contamination in the thin gate oxide. In any case, the process

only promises 8MHz so there was little to regret in failing to stretch

it to its limits.

$p

Appendix$ 1 contains brief details of this design.

$b2$v5

$l1uh

2.  Version Two...

$p

In the Autumn of 1982 a second design was begun of a 4-bit DtoA converter.

It was an ambitious scheme in which binary code to unary code conversion

would be performed synchronously by a depth$ 4 binary tree of cells

arranged as a 4-stage pipeline.

The resulting design, though conceptually elegant, was large and cumbersome

to lay out. The binary to unary conversion tree, for example, occupied

some 1800u by 800u and was entirely constructed from restoring logic.

Even the usually optimistic back of the envelope performance calculations

cast doubt on whether such a scheme would be viable at speed. Fortunately,

lack of time precluded the completion of this design.

A logic diagram of the system is included in Appendix$ 2;

$b2$v5

3.  Version Three...

$p

In the Spring of 1983 a third design, based on the second, was started.

The idea here was to use a 4 X 2^4 array of single transistor cells

to implement a steering logic solution to binary to unary conversion.

The unary outputs from the steering logic would be restored to respectable

levels, gated with a clock signal (to remove glitches) and then amplified

to drive the gates of 15 individually sized current sources.

Back of the envelope timing calculations indicated that the design was on.

$p

We started by designing the current sources. This time the difference between

drawn and fabricated channel sizes was allowed for in the program which

solved the Schichman-Hodges equations. For simplicity, a single enhancement

MOSFET between Vdd (5V) and the analogue output was used.  It is appreciated

that this will suffer significant stability and threshold variation problems,

however the solution is very simple.  More sophisticated switched current

sources could be used instead without altering the rest of the design.

Despite rounding the width of each current source to the nearest lambda

(3u) a SPICE simulation of the complete 15$ MOSFET unary to analogue

converter showed remarkable linearity over 0-1.1V. Provided that the

output range of the fabricated device can be tuned to this range by varying

the substrate bias (and hence the MOSFET threshold voltages), this linearity

should be preserved.

$p

Next, the clocked output drivers were designed. These sit between the

binary to unary conversion array and the unary to analogue converter.

Several iterations through SPICE were required in order to obtain the

required behaviour at 12.5MHz with a safety margin of a factor of 2.

The resulting inverter ratios look low by Mead and Conway standards,

but the waveforms predicted by SPICE2G.5 level-2 MOSFET models look

very convincing. Only fabrication will determine the correctness of these

predictions.

$p

Finally, the pad to bit-line (and clock-line) drivers were designed.

Again, several iterations through SPICE were required before satisfactory

output waveforms were observed.

$p

SPICE simulation would indicate that the most critical path in the

design is capable of responding to a 5ns rise-time edge (Schottky TTL

output) in under 20ns, a factor of 2 safety margin on a 40ns phase time

and 80ns clock-cycle time.  The design was submitted for fabrication in

June$ 1983 and we are hopeful of meeting our performance target.

$p

Appendix$ 3 contains full details of this design.

$n

$a sectno=1;pageno=1

$l1m

Appendix 1

$b2

$l1uh

The First Version

$p

The first version of the DtoA converter is written up in a separate

volume of rough notes and plots.

$n

$a sectno=2;pageno=1

$l1m

Appendix 2

$b2

$l1uh

The Pipelined Binary To Unary Tree

$p

This appendix gives brief details of the restoring logic, pipelined,

binary to unary tree scheme. This scheme was abandonned after a trial

layout because of its size.

$p

Firstly, there is a sketch of the system floorplan. Note that each cell

must be stretchable in the X-direction. This is particularly troublesome

for the input inverters at the top of the tree as they must then drive

very long lines.

$p

Secondly, there is a sketch of the tree cell. Note that it is a 2-phase

pipeline device.

$p

Finally, there is a plot of (the maximally compacted) layout of a tree cell.

Above the bottom level of the tree this cell must be stretched considerably

in the X-direction. For this reason, all signal lines run horizontally

in metal (so that metal, rather than poly or diffusion, is stretched out).

$p

The behaviour of the cell was never verified by SPICE simulation, however, a

complete switch-level simulation was performed using MIXSIM. This led to

the detection of a logic error in the clock phasing of the input delay

elements.

$n

$a sectno=3;pageno=1

$l1m

Appendix 3

$b

$l1uh

Calculating the Sizes of the Switched Current Sources

$b12

$l1um

The Unary to Analogue Converter

$b3

$l0

PROGRAM sjrc(input,output);

{A program to calculate the widths of 15 Enh MOSFETS designed

to produce a linear voltage output in the range 0-1V.  Each

MOSFET has its drain tied to Vdd (5V) and its source tied to

the analogue output (V). The gate is driven below threshold

(input 0) or to 5V (input 1).                               }

VAR     V : real;   {The desired output voltage}

rho : real;   {Width/Length of the MOSFET current source}

W : real;   {Width in microns of the MOSFET}

Wcum : real;   {Cumulative Width, in microns, so far}

lambda : real;   {currently 3u}

L : real;   {channel Length - currently 6u}

Vth : real;   {Threshold voltage - nominally 0.75V}

K : real;   {Transconductance in A/V**2}

n : integer;

BEGIN

lambda := 3;

L := 6;

Vth := 0.75;

Wcum := 0.0;      {cumulative width so far...}

K := 30E-6;       {transconductance}

FOR n := 1 TO 15 DO

BEGIN

V := n / 15;  {desired output voltage}

{now solve Schichman-Hodges}

rho := V * 2.0  / (75.0 * K * Sqr(5.0 - Vth - V));

W := rho * L / lambda;     {target width in microns}

writeln('M',n:1,'  17 ',n:1,'  18  16  MENH2 L=',L:4:1,

{and the increment, in multiples of lambda, required to

achieve the target width}

'U  W=',ROUND(W-Wcum)*3-3:1,'U');

Wcum := W;

END;

END.

$n

$l1uh

SPICE Input Produced by Program sjsrc

$b

The output produced by PROGRAM sjsrc is integrated with the rest of

a SPICE deck to give a complete description of the unary to analogue

converter sketched on the previous page.

$b

$l0

$.OPTIONS VNTOL=100U ABSTOL=1N LIMPTS=2801 ITL5=20000

$.WIDTH OUT=80

*

M1  17 1  18  16  MENH2 L= 6.0U  W=18U

M2  17 2  18  16  MENH2 L= 6.0U  W=18U

M3  17 3  18  16  MENH2 L= 6.0U  W=21U

M4  17 4  18  16  MENH2 L= 6.0U  W=21U

M5  17 5  18  16  MENH2 L= 6.0U  W=24U

M6  17 6  18  16  MENH2 L= 6.0U  W=24U

M7  17 7  18  16  MENH2 L= 6.0U  W=27U

M8  17 8  18  16  MENH2 L= 6.0U  W=30U

M9  17 9  18  16  MENH2 L= 6.0U  W=30U

M10  17 10  18  16  MENH2 L= 6.0U  W=33U

M11  17 11  18  16  MENH2 L= 6.0U  W=36U

M12  17 12  18  16  MENH2 L= 6.0U  W=39U

M13  17 13  18  16  MENH2 L= 6.0U  W=42U

M14  17 14  18  16  MENH2 L= 6.0U  W=45U

M15  17 15  18  16  MENH2 L= 6.0U  W=48U

* Drive into a load of 75 Ohms and 20pF

RLOAD 18 0 75

CLOAD 18 0 20PF

* Power and bias

VDD  17 0  DC 5

VB   16 0  DC -2.5

* V1-V15 turn on with 5ns rise times at 100ns intervals

V1   1 0 PULSE(0 5    0NS 10NS 10NS 2000NS 4000NS)

V2   2 0 PULSE(0 5  100NS 10NS 10NS 2000NS 4000NS)

V3   3 0 PULSE(0 5  200NS 10NS 10NS 2000NS 4000NS)

V4   4 0 PULSE(0 5  300NS 10NS 10NS 2000NS 4000NS)

V5   5 0 PULSE(0 5  400NS 10NS 10NS 2000NS 4000NS)

V6   6 0 PULSE(0 5  500NS 10NS 10NS 2000NS 4000NS)

V7   7 0 PULSE(0 5  600NS 10NS 10NS 2000NS 4000NS)

V8   8 0 PULSE(0 5  700NS 10NS 10NS 2000NS 4000NS)

V9   9 0 PULSE(0 5  800NS 10NS 10NS 2000NS 4000NS)

V10 10 0 PULSE(0 5  900NS 10NS 10NS 2000NS 4000NS)

V11 11 0 PULSE(0 5 1000NS 10NS 10NS 2000NS 4000NS)

V12 12 0 PULSE(0 5 1100NS 10NS 10NS 2000NS 4000NS)

V13 13 0 PULSE(0 5 1200NS 10NS 10NS 2000NS 4000NS)

V14 14 0 PULSE(0 5 1300NS 10NS 10NS 2000NS 4000NS)

V15 15 0 PULSE(0 5 1400NS 10NS 10NS 2000NS 4000NS)

*

$.TRAN  2.5NS  1500NS

$.PRINT TRAN V(18)

*

**********  ENHANCEMENT LEVEL 2  **********

*

$.MODEL MENH2 NMOS (LEVEL=2 VTO=0.75 GAMMA=0.46

+CGSO=4.5E-10 CGDO=4.5E-10 CJ=1.0E-4 CJSW=1.0E-9 JS=1.0E-7

+TOX=8E-8 NSUB=8.5E14 NFS=1E10 XJ=1.5U LD=1.25U UO=700

+UEXP=0.1 UTRA=0.3 VMAX=5E4 NEFF=3.0 XQC=0.4 DELTA=1.0)

*

$.END

$n

$l1uh

An Extract from the SPICE Output

$b

$l0

Digital Input (n)    Analogue Output (V)

1                7.561E-02 V

2                1.463E-01 V

3                2.235E-01 V

4                2.955E-01 V

5                3.723E-01 V

6                4.438E-01 V

7                5.188E-01 V

8                5.962E-01 V

9                6.681E-01 V

10                7.415E-01 V

11                8.158E-01 V

12                8.903E-01 V

13                9.644E-01 V

14                1.038E+00 V

15                1.110E+00 V

$b2

$l1uh

Least Squares Fit and Deviation from It

$b

The least squares linear approximation to these values is:

$b$l1

V = 0.074107 n  +  0.000488

$b

The deviation of the output from the least squares value, for each n, is:

$b

$l0

1               +0.001015 V       +1.3610$%

2               -0.002401 V       -1.6149$%

3               +0.000692 V       +0.3106$%

4               -0.001414 V       -0.4764$%

5               +0.001279 V       +0.3447$%

6               -0.001328 V       -0.2982$%

7               -0.000434 V       -0.0836$%

8               +0.002859 V       +0.4819$%

9               +0.000653 V       +0.0978$%

10               -0.000054 V       -0.0073$%

11               +0.000140 V       +0.0171$%

12               +0.000533 V       +0.0599$%

13               +0.000526 V       +0.0546$%

14               +0.000020 V       +0.0019$%

15               -0.002087 V       -0.1876$%

$b2

A plot of the spice output is shown on the next page. Note that the

current sources are driven by 5ns ramps from 0-5V at 100ns intervals.

$n

$n

$l1uh

Layout of the DtoA Converter

$p

The following figures show the layout and, where appropriate, the logic

of the important fragments of the design.

$p

Firstly, part of the binary to unary conversion array is shown. The four

metal lines are, from the left, Ground, most significant bit, Ground and

least significant bit. To make a 2-bit converter the diffusion lines on the

right should be attached to Ground and the diffusion lines on the left

attached to suitably sized pullups. The unary significance of these diffusion

lines increases from the bottom: the lower line is always 0; the next line

is 0 (asserted) if either of the bit lines are high; the 3rd and 4th lines

can only be 0 if the MSB line is high, with the 4th line being 0 iff both

lines are high. This scheme extends recursively to any number of bits

(but at an exponential cost in area!).

$p

Following this is a transistor diagram of the unary output clocked

amplifer. This includes the pullup alluded to above. The performance

of this circuit has been verified by SPICE simulation under the conditions

of being driven by the most critical unary output (the most significant

output, in fact) and driving the largest current source. Under these

circumstances there is still a factor of 2 safety margin. A plot of the

salient waveforms predicted by SPICE follows, followed by the layout of

a pair of these amplifiers (inputs at the top in diffusion).

$p

There follows a transistor diagram of the bit-line drivers used to

interface between the input pads and the bit lines which run across

the binary to unary conversion array. The performance of this circuit

has been verified by SPICE simulation and a plot of the salient waveforms

follows. Immediately following this is the layout of the bit-line driver

(input at the top in diffusion, output at the bottom in metal).

$p

Following this group of diagrams are 3 plots.

$p

The first is of the skeleton

of a pair or switched current sources. Inputs are on the right in diffusion;

Power is at the top and the bottom in metal; and the analogue output is

centre and left in metal. To 'program' this structure, enhancement mode

MOSFETS of an appropriate size (see start of Appendix$ 3) must be inseted

between the power rails and the analogue output rail with their gates

abutting the buried contacts shown on the right.

$p

The next 2 plots show the pads and wiring of the chip and the complete

chip respectively.

$p

Finally, there is a sketch of the proposed test rig and driving waveforms.