!{Blowing EPROMs on the Stag PPZ/Rainer Thonnes}

!<Transferring prom files between APM and Stag PPZ "universal" programmer

Files are normally prepared in Motorola S-record loader format, as generated

by most local assemblers.  There exist utilities for converting between

this format and pure binary (TOBIN and FROMBIN).  Alternatively, prepare

files in any format you know the Stag PPZ can cope with.

In what follows "NNN" means "not normally necessary".  This is because

the PPZ remembers all selected characteristics even when switched off.

So most parameters will normally already be set up as you will want them

to be.  (If you change obscure parameters to exotic values, please

change them back when you have finished, do NOT assume that when you

switch off they will revert to a default - they won't!)

!{\tt}

*  Ensure the PPZ is set up as follows:

- (NNN) Select "Set System" menu.

- (NNN) Select "Set Interface Hardware" sub-menu.

- (NNN) For each of "Serial I" and "Serial II", ensure the remaining

characteristics are compatible with APM and terminal (NB they

should be the same).  The normal values are: 9600 baud, 8 bits,

parity off, 1 stop bit.

- (NNN) Select "Set I/O Software" sub-menu.

- (NNN) Select format "MOTOROLA S-Record", and "Ctrl Z" OFF.

- (NNN) Select "Miscellaneous" sub-menu.

- (NNN) "Pass-through" ON.

* (NNN) Find the RS232 cable connecting the APM to the terminal.  Unplug

the terminal end and plug it into the connector on the back of the

PPZ marked "RS232(1)".

*  (NNN) Find a spare RS232 cable (straight-through, male-to-female), and

connect it between the terminal and the connector at the back of the PPZ

marked "RS232(2)".

*  Select "I/O" menu.

- (NNN) Select "I/O type" INPUT if you wish to send a file TO the PPZ.

- Select OUTPUT if you wish to send a file FROM the PPZ to the APM.

- (NNN) In either case, select "I/O Interface" SERIAL I.

- (NNN) If appropriate, modify the offset address and address limits.

- Select "Start I/O" function.

The PPZ screen should display the message

"Motorola S-record load" or "Motorola S-record dump".

The terminal should now behave as if it were directly connected

to the APM.  You may then log on and type the command "SETUP STAG".

*  Type the command  "ZSEND X"  or  "ZREAD /Y"  depending on whether you

wish, respectively, to send a file to the PPZ or read a file from the

PPZ, with X and Y being the names of the files involved.

Once the command has been typed, the message on the PPZ screen should

change to "Motorola S-record load/dump in progress", and the

terminal WILL GO DEAD.

When the transfer has finished, the message should change to

"Motorola S-record load/dump completed", but the terminal will REMAIN

DEAD until you select "RESET" or "Start I/O" again.

!{\rm}

!>

RWT January 1986 revised March 1987