Edinburgh University Computer Science Department

Motorola 88100 Assembler User Notes

This document relates to a locally written  assembler  for  the  Motorola  88100

processor.    It   describes  the  input  language  accepted,  as  well  as  the

organisation of the output object file produced.   The assembler is designed  to

be  used in conjunction with high-level language compilers not yet written.   In

particular this means that there is support for  external  linkage,  i.e.  cross

references  (both  to  code  and  data  objects)  between separately compiled or

assembled modules.  It also means that each separately compiled module is likely

to consist of two areas, one containing code, the other static data and external

linkage information.   It is assumed that at run time this  data  area  will  be

pointed at by a base register.   One of the registers "reserved for the linker",

R29, has been commandeered for this purpose, and has been called SB  for  Static

Base.

To  supplement the assembler, a linking loader is available, which takes several

separately assembled (or compiled) object files and combines them into a  single

image suitable for loading into EPROM.  This is also described.

The  user  (assembly  language  programmer)  is expected to be familiar with the

architecture and instruction set of the processor,  and/or  to  have  access  to

Motorola's "MC88100 RISC Microprocessor User's Manual".

This  is  a  one-pass  assembler.   Therefore the listing file which it produces

while processing the source input cannot show the  actual  branch  displacements

for forward references.  The generated binary code is therefore omitted from the

listing  altogether,  which  as  a  result  contains  only line numbers, address

offsets, source text, and error messages.   The generated code is not output  on

the  fly,  but buffered internally to allow forward references to be fixed up as

soon as the relevant labels are reached.   As an option, an  additional  listing

file  may  be  generated from the buffered code, which consists of code address,

code value, and disassembled code, but without the original source text or  line

numbers.   This  feature  is particularly useful in the development stage of the

assembler, as it may be used to check that it is working correctly.

Using the assembler

The assembler is available on the APMs, in directory ASSEM on filestores  B  and

C.  The command to invoke it is:

ASSEM:ASS88K file {-{NO}OBJ} {-{NO}LIS} {-{NO}DIS}

The  single  parameter  is  the  name of the file to be assembled.   A file name

extension of ".88K" is assumed, and the output  will  go  to  three  files  with

extensions ".OBJ", ".LIS", and ".DIS", respectively.   The options -OBJ, -NOOBJ,

-LIS, -NOLIS, -DIS, -NODIS (which may be abbreviated to -O etc) may be  used  to

enable or disable generation of these respective files.  The default options are

-OBJ-LIS-NODIS.

Using the loader

The  loader  will  link  together a number of object modules, and build an image

file suitable for putting into EPROM.  The command is:

ASSEM:LOAD88K  infile1, infile2, ... / outfile {-codebase=??} {-database=??}

The input file names are all assumed to have extension ".OBJ", the  output  file

will have extension ".ROM".  If "/outfile" is omitted, the output file will have

the same name as the first input file.   Specification of the code and data area

base addresses is optional (they default to 0 and the size  of  the  code  area,

respectively), their values are used in the computation of addresses of external

objects.  The "??" values are specified in hexadecimal.

The  layout of the output image is simply the juxtaposition of the code areas of

all the input files (in the order in which they were specified in the  command),

followed  by the similarly juxtaposed data areas, in the same order, initialised

as specified in  the  respective  object  files,  with  all  links  required  by

import/export directives filled in appropriately.

If  it  is sufficient for the data areas to be read-only, the above defaults are

satisfactory.   Normally, however,  the  programmer  will  expect  these  to  be

writeable,  so  a  non-zero  value  for  the  base  address  of  the  data  area

(corresponding to writeable RAM in the  address  space  of  the  hardware)  will

almost always need to be supplied.  Being ROM based code, it is assumed that the

main (first) file will begin with an exception vector table.   The first word of

this contains the  first  instruction  to  be  executed,  and  the  loader  will

overwrite  the  second word with two halfwords, namely the sums of the sizes (in

words, not bytes) of the code and data areas, respectively,  of  all  the  files

being linked together.

It  is  recommended that the main program begins by copying the ROM image of the

data areas into RAM.  To do this, make the first instruction of the vector table

a BSR to the start of the main program, this points R1 at the two aforementioned

halfwords.   It may be a good idea to make the first word of the  data  area  an

external  self-reference,  in  order to convey the RAM area start address to the

program.   The alternative of having that value specified as a constant  in  the

program  involves  the risk of it becoming inconsistent with the value passed as

parameter to the linker.  Example:

BSR COPY            ; first word of vector table

PLANT 0             ; dummy second word

...                 ; rest of exception vector table

SELF DATAEXPORT SELF     ; export start of data area

DATAIMPORT SELF     ; and import it again

COPY LD.HU T1,0(R1)      ; T1 = size of code area

LD.HU T2,2(R1)      ; T2 = size of data area

SUBU R1,R1,4        ; point R1 at start of code area

LDA SB,R1[T1]       ; point SB at start of ROM data area

LD T1,SELF          ; point T1 at start of RAM data area

LOOP SUBU T2,T2,1        ; decrement word count

LD T3,SB[T2]        ; copy using count as index,

ST T3,T1[T2]        ;   i.e. copy last word first

BCND GT0,T2,LOOP    ; go round for more until T2=0

Pre-defined operand names

The assembler knows the names of the general  registers  (R0-R31),  of  the  CPU

control  registers  (CR0-CR20),  and  of  the  FPU control registers (FCR0-FCR8,

FCR62, FCR63).   Bearing in  mind  conventions  suggested  in  the  manual,  the

assembler recognises alternative names for the general registers, as follows:

P1-P8     R2-R9     Eight registers for passing PARAMETERs to procedures

T1-T4     R13-R10   Four TEMPORARY working registers

V1-V12    R14-R25   Twelve registers to be used as local VARIABLES

B1-B2     R27-R26   Two local BASE registers

PB        R28       The PROGRAM base pointer

SB        R29       The STATIC base pointer

FP        R30       The FRAME pointer

SP        R31       The STACK pointer

Regarding  R26-R29,  the  words  "reserved for linker" in the manual are loosely

interpreted here as "base registers  for  various  purposes  including  external

linkage  between  separately compiled modules".   The use of SB has already been

explained.   L1 and L2 may be used by compilers  to  access  intermediate  level

variables,  for high-level languages which allow this (Imp and Pascal do, C, not

being high-level, does not), PB is a spare "Global Data Pointer" register.   The

exact  purpose  of  L1,  L2,  and  PB is left open to be defined by the compiler

designer.

The  following  alternative  names  for  CPU  and  FPU  control  registers   are

recognised, and the registers they refer to are those stated in the manual.

PID, PSR, EPSR, SSBR, SXIP, SNIP, SFIP, VBR, DMT0, DMD0, DMA0, DMT1, DMD1, DMA1,

DMT2,  DMD2,  DMA2, SR0, SR1, SR2, SR3, FPECR, FPHS1, FPLS1, FPHS2, FPLS2, FPPT,

FPRH, FPRL, FPIT, FPSR, FPCR.

Various  constants  are  also  pre-defined,  especially  for  use   in   BB0/BB1

instructions  following a comparison or for use in the BCND instruction.   These

constants are:

For BCND: GT0, LE0, EQ0, NE0, GE0, LT0 (refer to BCND instruction in manual).

For BB0/BB1: NC, CP, EQ, NE, GT, LE, LT, GE, HI, LS, LO,  HS,  OU,  IB,  IN,  OB

(refer to CMP and FCMP instructions in manual for meanings).

General syntax

The assembler expects statements at most one per line.  Comments may be included

in  the  source  text  in one of two forms.   They either begin with a semicolon

(';') and extend to the end of the line, or they begin  and  end  with  matching

curly brackets, in which case they may extend over several lines.

Statements  consist  of a label, an instruction, and parameters.   The label and

instruction are optional, and the number of parameters required depends  on  the

instruction,  if  present.   A label, if present, must start at the beginning of

the line, and may be followed by a colon (':').   Instructions may not  commence

at  the  beginning of a line, i.e. if they are not labelled, the line must begin

with at least one space.

An "instruction" can take one of two forms.   It is either  a  real  instruction

(i.e.  an  88100  opcode)  or an assembler directive.   If a real instruction is

labelled, this has the effect of defining that label to be the address  of  that

instruction.   This  effect  does  not pertain to labels attached to directives.

Each particular directive defines its own  effect  on  its  label,  indeed  some

directives require a mandatory label, while some require to have none.   A label

without an instruction has the same effect as a label with a  real  instruction,

i.e.  it  defines  the  label  as  being  the value of the current code location

counter, which is the address of the next instruction.

Names (of instructions, labels, and operands) consist of any number of (well, at

most 255) letters, digits, underline, or dollar symbols, but must begin  with  a

letter.  Upper and lower case letters are not distinguished.  The special name *

may  be  used  to  refer  to  the  address of the current instruction.   Literal

constants consist of signed decimal numbers or  quoted  ASCII  constants  (using

single quote marks, e.g. 'A', '?', or '''' (note special form of quoted quote)).

Non-decimal  numbers are specified using the form RADIX_NUMBER; any radix may be

used, but the most useful will be hexadecimal, octal, or binary  (e.g.  16_FF80,

8_507, 2_10101).   An alternative notation is also accepted (e.g. 0xff80, 0o507,

0b10101).   Single-precision floating-point constants are recognised, and  these

take the form

<integer-part>.<fraction-part><optional-exponent-part>

Note  that  as the integer-part is mandatory, .1 must be expressed as 0.1.   The

optional exponent part consists of '@' or  'E'  or  'e'  followed  by  a  signed

integer.

Constant  expressions  may be formed by stringing together literal or predefined

integer constants, using the usual arithmetic operators.   Note,  however,  that

the  assembler  does  not  observe  the usual rules of operator precedence.   It

evaluates expressions strictly  right-to-left,  but  brackets  may  be  used  to

over-ride  this.   The operators recognised are: +, -, *, /, %, \, &, !, !!, <<,

>> (% means remainder, \ means exponentiation, ! means or,  !!  means  exclusive

or).   All  these  operate on integer expressions only.   Although the assembler

accepts floating-point numbers in this context, they will be treated as  if  the

IEEE  pattern  were  integers,  and  so  the  effect  may or may not be what the

programmer  expects.  "1.5+1",  for  example,   is   pretty   meaningless,   but

"1.5&16_ffff"  and  "1.5>>16"  may be used to load (or or) the two halves of the

number 1.5 into a register.

Syntax of memory operands

The memory referencing instructions (XMEM, LD, ST, and LDA) require one of their

parameters to be a register (the one that will be loaded from or  stored  to  or

have  its  contents  exchanged  with memory), the other parameter identifies the

memory operand.   The manual shows three notations for this,  depending  on  the

addressing  mode.   For  scaled indexing the notation places the index register,

enclosed in square brackets, after the base register (as in ST T1,FP[T2]), while

for unscaled and immediate indexing  the  convention  is  to  place  the  index,

whether  it  is  a constant or a register, after the base register, with a comma

inbetween (as in LD T1,FP,T2 or ST T2,SP,8).   While this assembler accepts this

comma-form, it also allows the form in which the index is placed before the base

register,  the  latter  being  enclosed in round brackets (as in LD T1,T2(FP) or

ST T1,8(SP)).

Furthermore, it is possible  to  define  a  name  as  being  a  complete  memory

reference,  using  the  DEF,  DS,  or  VAR  directives.   For example, following

"FRED DEF R5[R6]"  and  "BERT DEF 4(FP)",  one  could  write  "LD T1,FRED"   and

"ST T1,BERT", which would mean the same as "LD T1,R5[R6]" and "ST T1,FP,4".

A few notes regarding some instructions

All  the  instructions  mentioned  in  the manual are recognised.   In addition,

alternative mnemonics ASR and LSR are accepted for EXT and  EXTU,  respectively,

and  alternatives  ASL  and  LSL are accepted for MAK.   Remember that ROT means

rotate RIGHT.

Unlike as recommended in the manual, the assembler does not convert LDA to  ADDU

in  circumstances where this would be preferable, this responsibility is left to

the programmer.

Assembler Directives

BEGIN and END may be used to restrict the scope of labels  and  names,  so  that

these may be re-used.  BEGIN/END may be nested to any depth.

INCLUDE  may be used to incorporate the contents of another file into the source

text.  The parameter should be a filename enclosed in quotes.

DEF is used to define a name to be either a constant or an operand reference.  A

label is mandatory.  Examples: "LIMIT DEF 127" or "SP DEF R31" or "I DEF 8(FP)".

EQU is accepted as an alternative to DEF.   A warning is given if an attempt  is

made to define a name which already exists.   REDEF is provided for occasions on

which it is desired to do this.

DC (with optional size suffix .B, .H, .W, .F, or .D) stands for DEFINE CONSTANT,

and is used to plant a value (byte, halfword, or word  (used  if  no  suffix  is

given), and single and double precision floating point) into the data area.   If

a label is present, that name is defined as a memory reference relative to  base

register  SB, at the current data offset, which is first aligned to the relevant

multiple of 2, 4, or 8 if necessary.  Then the value which follows as a constant

expression is added to the data area, and the offset is incremented by its size.

A list of values separated by commas may be given.   If, in the case of DC.B, it

is  desired  to  plant  a  text  string, a list of single characters enclosed in

quotes may be abbreviated (e.g. 'a','b','c' may be written as 'abc').  If double

quotes are used instead, the string length byte is planted first (e.g. "abc"  is

equivalent to 3,'a','b','c').

DS  (with  optional suffix .B, .H, .W, .F, or .D) stands for DEFINE STORAGE, and

is used to reserve space in the data area without planting initial values.  If a

label is given, it is defined, as with DC, as a memory reference into  the  data

area.   The  optional  parameter is a constant expression denoting the number of

slots (of the size determined by the suffix) to  reserve,  i.e.  the  amount  by

which  to advance the data offset.   A value of 1 is assumed in the absence of a

parameter.

The BASE directive is used to establish the base register and initial offset for

subsequent use of the VAR directive, below.

The VAR directive is similar to the DS directive in that it reserves  space  and

defines  the  label,  if  any,  as  being a memory reference at a certain offset

(which is incremented appropriately) from some base register.  Example:

BASE 8(FP)     ; establish current base as 8(FP)

I    VAR            ; define I as 8(FP), increase offset by 4

J    VAR 3          ; define J as 12(FP), inc by 12

K    VAR.H          ; define K as 24(FP), inc by 2

L    VAR.H          ; define L as 26(FP), inc by 2

The following four  directives  are  concerned  with  external  linkage  between

modules.  The cases numbered 1-4 are discussed below.

label  CODEIMPORT "name"  ; case 1

label  CODEEXPORT "name"  ; case 2

label  DATAIMPORT "name"  ; case 3

label  DATAEXPORT "name"  ; case 4

Cases  2  and 4 define "name" as an object defined in (i.e. to be exported from)

the module being assembled, case 2 exports an external procedure from  the  code

area, at the current location counter value, case 4 exports an external variable

from  the  data area, at the current data offset.   Labels, if present (and they

usually will be), are defined in the normal way as a code label (case  2)  or  a

data reference as with the DC/DS directives (case 4).

Cases  1  and  3 assert that "name" is an object defined in (exported from) some

other module (to be imported into this one).   In both cases the labels (usually

present) are defined as a data reference, and in both cases the effect is to add

a  word  (or  two)  to  the  data area.   In case 3 one word, the address of the

external object, is added.   In case 1 two words are added, firstly the  address

of  the  entry  point of the external procedure, and secondly the address of the

beginning of the data area belonging to the module  containing  that  procedure.

This is needed because the external procedure, once called, may not only need to

access static variables of its own, but may indeed need to call further external

procedures,  the  addresses  of which will be stored somewhere in its data area.

We assume that the called procedure has no way of working  out  for  itself  the

address of its data area, and therefore it has to be communicated in this way by

the  calling  procedure.   It is the responsibility of the loader to fill in the

correct addresses, and the object file contains all the information needed to do

this.  A typical call sequence to an external procedure might look like this:

FRED  CODEIMPORT  "BERT"

{evaluate and pass parameters as needed}

LD R1,FRED         ; pick up addr of entry point

JSR.N R1           ; call procedure, but first

LD T1,FRED+4       ; establish new data pointer

PLANT may used to generate a value  in  the  instruction  stream  that  can  not

otherwise  be  generated  in  the  normal  way.   It  is  followed by a constant

expression, which evaluates to a full word.   Any label is treated as  a  normal

code label.

Layout of object code file

The  object  file  consists of a two-part header followed by several information

sections.   The first part of the header contains simple items  of  information,

while  the  second  is a list of sizes of the information sections which follow.

The layout of the header is somewhat future-proof in that extra  fields  may  be

added  at a later date without disturbing the meaning of those which are already

defined.   This is achieved by incorporating information about the size  of  the

header  parts  into  the first word of the header.   Throughout the object file,

multi-byte values are presented in big-endian order, i.e. most significant  byte

first.

At  present  only  two words are defined for the first part of the header.   The

first word contains two 8-bit bytes giving the  sizes  (in  words)  of  the  two

header parts, and a 16-bit magic number, the purpose of which is to identify the

file  as an 88100 object module, its value is 88100 modulo 65536.   As the first

and second parts of the header are currently defined  to  contain  two  and  six

words, respectively, the value of the first word of the object file is therefore

16_02065824.   The  second  word  is  the size of this module's static data area

(needed by the loader when it allocates space).

The second part of the header contains the  sizes  of  the  six  sections  which

follow.   The  first four sections contain, respectively, information about code

imports, code exports, data imports, data exports.   The fifth section  contains

the  module's  code,  and  the  sixth  contains information for initialising its

static data area.

The four external linkage sections, not all of which need be present,  all  have

the same layout.  For each external name in the section, there appears an offset

word  (even  though data area offsets are limited to two bytes), followed by the

name itself (prefixed by a length byte and padded out at the  end  so  that  the

length  byte  plus the characters of the name plus the padding occupy a multiple

of four bytes).  The offset defines where the object being exported is, or where

in the data area the loader should fill in the address information  of  imported

objects.   The  list  is terminated by a dummy entry in which the name string is

null.

The code section simply consists of the code to be loaded.

The data initialisation section consists of a number  of  segments  (this  saves

object  file  space when there are significantly large uninitialised portions in

the data area).   Each segment begins with a size word (number of bytes),  which

is  followed  by an offset word, which is followed by the data.   The segment is

padded out to a multiple of four bytes, and there is a dummy null segment at the

end (consisting just of one zero word).

RWT December 1989, revised 20/12/90 and 25/04/91