!{APM Cross-Assembler for M6800/Rainer Thonnes}

EUCSD Motorola M6800 Assembler  -  User Notes

This assembler is a local implementation of the  Motorola  M6800  cross

assembler,  documented  in  the 'M6800 Microprocessor Programming Manual'.

Wherever possible it is upward compatible with the Motorola assembler  and

like it, contains no macro facilites.

!<Differences

!<Restrictions

!{\tt}

1) Only one program may be assembled in a single input stream.  As a

result the MON and END directives are synonymous and optional.

2) Only object tapes are produced by the assembler.  As a result the

OPT  directive  options  MEMORY  and OTAPE and their variants are

illegal.

3) Error messages are of a different and fixed form  and  hence  the

OPT directive options SERROR and SER are synonymous with ERROR.

!{\rm}

!>

!<Extensions

!{\tt}

1)  The  mnemonics  LSL, BHS and BLO are provided and are synonymous

with ASL, BCC and BCS respectively.

2) The delimited form of  the  FCC  directive  accepts  the  doubled

delimiter  as a single instance of the delimiter in the generated

text.

e.g.                     FCC   /A//B/

is the same as           FCC   'A/B'

3) The OPT directive has the following additional options:

Long form  Short form

BLANK      B            Ignores  redundant  spaces   in   operand

field. e.g. around ',' and '+'.

NOBLANK    NOB          No   spaces   allowed  in  operand  field

(selected by default).

CYCLE      C            Print execution time  in  machine  cycles

for each executable statement.

NOCYCLE    NOC          No  execution  times printed (selected by

default).

FULL       F            Allows  fully  parenthesised   arithmetic

expressions as operands (see (4) below).

NOFULL     NOF          Only      unparenthesised     expressions

evaluated strictly from left to right are

allowed   as   operands   (selected    by

default).

WRAP       W            Long input lines are wrapped around on to

next line of listing.

NOWRAP     NOW          Long input lines are truncated in listing

(selected by default).

XREF       X            Symbol  cross  reference is listed at end

of listing.

NOXREF     NOX          No  cross  reference  table  is   printed

(selected by default).

4)   By   specifying   OPT   FULL,  fully  parenthesised  arithmetic

expressions evaluated in the usual algebraic order are acceptable

as operands.  The valid binary operators are (in increasing order

of precedence):

! (or), \ (exclusive or)

& (and)

+, -

*, /, | (remainder)

< (left shift), > (right shift)

The unary  operators  +  (ignored),  -  and  \  (complement)  are

accepted and take precedence over all binary operators.

5) Apart from catastrophic failures, error messages are generated in

the  final  pass  only  and always immediately follow the line to

which they apply.

6) This is a multi-pass assembler in which there are no  constraints

on  the  order  of  definition  of  symbols and which attempts to

optimise the use of direct addressing.

!{\rm}

!>

!>

!<Implementations

!<LEGOS

assem.M6800 <source>/<object>,<listing>

eprom.m6800 <source>/<object>,<listing>

No default file names are provided.   The ASSEM version generates

object  code in Motorola M6800 loader format.   The EPROM version

produces a binary file suitable for the  SOFTY  EPROM  programmer

and M6802 personality cards.

!>

!<VAX

M6800

The  user  is  promted  for  i/o streams.   The object code is in

Motorola M6800 loader format.

!>