Programmer's Guide to the APM 82586-based Ethernet Interface

Mechanical Interface  (Please plug it in the right way round!)

The board has two 96-pin DIN connectors, one of which plugs into the "system

bus" (backplane), the other should be connected to the "front bus" (a ribbon

cable to the CPU board).  It is possible to get it wrong and thereby to damage

the board.  The backplane is used only to power the chips on the board (and the

tap), all communication with the CPU takes place over the front bus.  When

inserting the board into an APM, the wiring should be on the lower left and the

chips on the lower right (all APM boards have the chips on the right).  Handle

the board by gripping the upper half (where there are no chips and hence no

wiring) to avoid damaging the wiring.

Connection to the transceiver (tap) is by a 15-pin D-type socket on a short

length of ribbon cable plugged into a 16-pin chip socket.  This should be

plugged in such that the ribbon emerges from the DIL header plug in the

direction towards the back of the APM, i.e. away from the centre of the board.

Hardware interface as seen by programmer

The board contains the Intel 82586/82501 chip set, 128 kbytes of dual-ported

memory, and glue to make it work.  The on-board memory is used to communicate

between CPU and 82586, but it should be noted that the 82586 can access ONLY the

on-board memory, not the entire CPU's local address space. Contention between

82586 and CPU for this memory is resolved by on-board arbitration logic.

The memory appears at addresses 040000 to 05FFFF in the CPU address space.  Due

to the fact that the 82586's high-order 7 address bits are ignored, the memory

appears at X to X+01FFFF in the 82586's address space, where X is any multiple

of 020000.  The Channel Attention (CA) signal is generated as a side-effect of

the CPU reading from any location in the range 05FFF0 to 05FFFF.  Interrupts

from the 82586 to the CPU occur at priority level 4, and the CPU board is of

course configured for auto-vectoring.  Care is needed because the interrupt

vector concerned (stored in location 001070) is also used by the 2 MHz ether

board.

Byte-sex disparity

The 82586, designed to be used in conjunction with Intel processors, assumes an

addressing scheme in which the low-order bytes in 16-bit or 32-bit words appear

in that byte of the word which has the lower address.  This is the opposite to

what the Motorola CPU assumes.  This presented the board designer with a choice

(dilemma) between two approaches:  The 16-bit data buses of CPU and 82586 could

be connected either to achieve word-fidelity (a 16-bit word has the same meaning

seen from both sides) or byte-fidelity (consecutive bytes appear consecutively

on both sides).  The latter, which was chosen, limits the overheads of software

byte-swapping to manipulation of pointer and count fields in the 82586's data

structures.  The former would also have necessitated inverting address bit A1 to

achieve fidelity of 32-bit words.  To make sequential byte accesses appear right

would have meant exclusive-orring 3 into all byte addresses in software.

Software Interface (Driver)

A library of external variables and procedures is available which presents an

interface in which the tedia of interrupt handling, byte swapping, descriptor

manipulation, and buffer management are hidden from the (Imp) programmer.

To use this driver, your program should contain  '%INCLUDE "82586:DRIVER.INC"',

and before running your program (perhaps in your LOGIN.COM file), you should

give the command  'INSTALL 82586:DRIVER'.

As already mentioned, the 82586 cannot access data at arbitrary places in the

CPU address space, i.e. all descriptors and data buffers must reside in the

on-board dual-ported memory.  Presenting a software interface in which the

programmer supplies arbitrary buffers would involve substantial copying

overheads, so instead the driver makes on-board buffers available to the

programmer via data pointers.  The programmer is free either to move data

between these buffers and elsewhere or instead to generate or consume the data

in situ.  The driver looks after management of these buffers behind the scenes,

without the need for programmer concern.

Each packet ("frame" in Intel parlance) is associated with only a single data

buffer, i.e. the buffer chaining capability of the 82586 is not used.  All

packet buffers are large enough to contain the maximum permitted size of packet

(1500 bytes).

Starting it up, shutting it down, changing modes:

To activate the driver, the programmer should call the routine  ETHER STARTUP,

giving it as parameter an array indexed from 1 to 6, containing the DTE address

he wishes to be associated with this ethernet station.  Alternatively, a NIL

reference may be passed, in which case the driver generates a default value

(currently this value is  46 72 65 64 2D XX  where XX is the APM's address on

the 2 Mhz ether).  The routine supplies the 82586 with receive buffers, starts

it up in normal mode, and inserts an interrupt handler.  If the user program

crashes, the APM will need to be rebooted unless it has been possible first to

call the routine  ETHER SHUTDOWN, which resets the 82586 and removes the

interrupt handler again.  Prior to calling  ETHER STARTUP, the programmer may

wish to select abnormal operating modes by writing non-zero values into any of

the following configuration variables:

ETHER SAVBF   (Save Bad Frames - necessary if in loopback mode it is desired to

handle packets shorter than 46 bytes)

ETHER ATLOC   (Address and Type Location - if it is desired NOT to separate the

14-byte header from the 1500-byte data, if this is selected the

SOURCE and DESTINATION address arrays as well as the TYPE

variables will be meaningless, and the data buffers will be taken

to be 1514 bytes long instead of 1500)

ETHER INTLOOP (internal loopback mode)

ETHER EXTLOOP (external loopback mode)

ETHER PROM    (promiscuous mode)

ETHER FIFOLIM (this should take a value in the range 0 to 15 - default is 8)

Although as part of normal initialisation the startup routine calls the routines

ETHER LOCAL ADDRESS  and  ETHER SET MODES, the programmer may call these at any

stage after initialisation.  The former takes a DTE address array as parameter,

the latter has no parameters other than the above global variables.

Multicast addresses

In order to make the 82586 listen to a set of N multicast addresses, it is

necessary to call the routine  ETHER MULTICAST ADDRESS  N+1 times.  For each of

the first N times the parameter should be a DTE address array indexed from 1 to

6, the last call should have the NIL parameter.  The first N calls (N must be

less than 256) accumulate the addresses in a transmit buffer, the NIL call

submits the buffer to the 82586 and causes it to re-build its multicast filter.

Having called  ETHER MULTICAST ADDRESS  one or more times it is MANDATORY to

call it with a NIL parameter, otherwise the next transmit command will cause

trouble.

Transmitting packets

After initialisation, a transmit buffer is made ready, and the array pointers

ETHER TRANSMIT DATA (1:1500)  and  ETHER TRANSMIT DEST (1:6)  are pointed at the

appropriate part of the buffer and descriptors involved.  The programmer should

fill in the packet data and destination DTE address as required, and set the

variables  ETHER TRANSMIT TYPE  and  ETHER TRANSMIT SIZE  as well. The former is

the "Protocol Type" if XDI (Xerox/DEC/Intel) TCP/IP-style protocols are used,

for ISO/IEEE compatible protocols this field is used to indicate the size of the

data portion of the packet.  The latter is the physical size of the data portion

of the packet.  To achieve the minimum physical total packet size (of 64 bytes)

required by the Ethernet standard to prevent receivers dismissing the packet as

noise, this field should be at least 46 (after adding 14 bytes for the header

and 4 for the CRC, that makes 64), even if there is less actual information in

the packet.

Having filled in these two arrays and two scalar variables, the programmer

should call the routine  ETHER TRANSMIT FRAME, which causes the buffer to be

passed to the 82586, and another buffer to be made ready.  In other words, a

side-effect of calling the transmit routine is to point the two array pointers

to somewhere else (the next buffer) and to clear the TYPE and SIZE variables.

Receiving Packets

Before it is possible to receive anything, it is necessary to register an

interest in one or more particular protocol types or ranges of types.  Incoming

packets in which the type field does not fall into one of the registered ranges

are ignored.  Registering and un-registering an interest is done by calling the

routines  ETHER ACCEPT TYPE RANGE  and  ETHER IGNORE TYPE RANGE, both of which

take two parameters in the range 0 to 65535, the first being a lower, the second

an upper bound, both inclusive.  For example, to accept everything use 0 and

65535, to accept ISO/IEEE only use 0 and 1500, to accept only a single type such

as DEC's 16_6007, use the same value for both bounds.  Several different ranges

may be active together, but for best results they should not overlap, as there

is a danger of received packets becoming trapped, thereby reducing the size of

the receive buffer pool.  When calling IGNORE, both bounds must match exactly

those specified in the corresponding ACCEPT call.

The receiver operates independently of the user program.  Incoming packets are

stored on queues corresponding to registered type ranges.  The user program

causes packets to be removed from the queues, and eventually to be returned to

the free pool to be filled with further incoming packets.

Once an interest has been registered, the user program may call the predicate

(boolean function)  ETHER FRAME RECEIVED, which takes a single parameter, namely

a protocol type number.  The function searches the interest register until it

finds the most recently registered range into which the parameter falls.  The

least recently received packet, if any, which is stored on the queue for that

range is then made available to the user program, and a TRUE result is returned.

A FALSE result means either that the parameter is not in any of the registered

ranges or that the queue for the first range found is empty.

Making a received packet available to the user program is done by pointing the

three array pointers  ETHER RECEIVE DATA (1:1500), ETHER RECEIVE SOURCE (1:6),

and  ETHER RECEIVE DEST (1:6)  at the appropriate places in the buffer and

descriptor, as well as filling in the global variables  ETHER RECEIVE TYPE,

ETHER RECEIVE SIZE, and  ETHER RECEIVE STATUS.  The latter is zero for a packet

received without errors (but defective packets are only passed on to the user

program when  ETHER SAVBF  is non-zero, so unless this is the case, the user

program need not check this variable).  SIZE indicates the physical size of the

data portion of the received packet, i.e. it does not include the 14 byte header

(destination address, source address, and type) nor the 4 byte CRC.  The

received destination address is made available to the user program because it

may be the broadcast or a multicast address, though usually it will be the local

address established during startup.

A side-effect of calling  ETHER FRAME RECEIVED, even if the result is FALSE, is

to return the buffer of any previously received packet to the pool.  So it

should not be called until after the previous packet has been either consumed or

copied.

After startup and before calling  ETHER FRAME RECEIVED  for the first time,

ETHER RECEIVE DEST  points to a copy of the local DTE address, useful if this

was established by default (by passing NIL to ETHER STARTUP).

Statistics

A number of variables provide information on how often certain events occurred,

most are self-explanatory:

COMMANDS EXECUTED/FAILED

FRAMES RECEIVED/LOST/DISCARDED  (LOST due to faulty status, DISCARDED due to

lack of type range match,  RECEIVED is sum of

LOST, DISCARDED, and those stored in the

range queues)

EXTRA BUFFERS  (number of times a packet overflowed a buffer - this would

indicate that an over-size packet was heard, as all buffers

are big enough to hold the maximum size packet)

BUFFER/DESC LOW WATER  (the lowest number of receive buffers or descriptors

in the pool, initially both are 64)

TX/RX RESTARTS  (TX should remain 0, RX will count up each time the receiver

reaches the "no resources" state, i.e. LOW WATER will have

reached 0)

CRC/ALN/RSC/OVR ERRS  (copies of 82586 SCB error counts at last interrupt)

RWT 5/9/89