VISION:LIGHT  and LIGHTS                Gavin Brelstaff 20/2/85

Lights will run on a half mega-byte machine whereas light need a full MB.

RUNNING INSTRUCTIONS

light {-flag[value]} input_file output_file

Flags

-x<picture size>   picture size is the number of pixels along the square image.

-i<iterations>    iterations means the number of iterations to be used.

Default 2000.

-p<precision>     precision is the number of bits in addition to the 8 in

the input intensities, that will be used in aritmetic

calculations.

Default 16.

-j<value>        value is an integer in the range [1,16] by which the

relaxation parameter w is set:

w = 1 + value/16.

w is defined in the book by Young on page 72 and is related

to the lambda parameter used by Blake in his lightness paper

by

w = 1 -lambda.

Default value = 8.

-m             Causes the output intensity values to be in the range [0..256].

The default range is [-128..127] which is called the mean-zero

option which is compatible with the "lightness" files used by

the program called vision:land.

-l            Causes the input intensities not to be converted into logarithms.

The default is to convert.

-t<value>     value is a floating point number that defines the thresholding

value.   In the case of the log conversion option (not using -l)

the value represents the percentage change in intensity at

threshold.  Otherwise it is compared directly to the input

intensities which range from [0..256].

Default 6% or 100.

-b           Causes a beep sound after every ten iterations.

Default off.

-s           Supresses messages to screen. Useful for unwatched batch jobs.