SDA User Notes

SDA is a "Simple Design Aid" for the production of circuit boards on our

BEPI Solder-Wrap machine.  It lacks the sophistication of other design

suites such as PCAD or our in-house ESDL.  Its aim is to provide the

designer with a quick and easy means of generating the data necessary to

drive the wiring machine, without incurring substantial overheads of

learning how to use the design package.  You can learn how to use SDA in

minutes or hours rather than days or weeks.

What SDA does NOT do:

Produce pretty drawings.  The designer is expected to have the design

sketched out in his mind, or on the "back of an envelope".  Adequate

"working" drawings can be drawn by hand (on more envelopes) and

annotated with pin-numbers etc from lists produced by SDA.

Perform automatic subcomponent assignment, i.e. decide which gates to

group in which chips.  The designer is expected to work in units of

complete chips.

Perform automatic component placement.  An interactive semi-automatic

placement tool may be made available in due course, but in the meantime

the designer is expected to produce his own layout, and tell SDA where,

in terms of column and row numbers, he wants each chip to go.

What SDA expects:

The designer should produce a single input file, in the format shown

below, which contains the following information:

[A] The absolute coordinates of the columns and rows of component pins,

which should be arranged in almost-regularly spaced columns, usually

0.3" apart, and regularly spaced rows, always 0.1" apart.  This

information essentially describes the geometry of the particular blank

board being used, and is usually expressed in a single line of text.

[B] The relative coordinates of each pin of each type of package (eg DIL20)

used in the design, usually expressed relative to the position of pin 1.

A simple dual-in-line package is usually described in a single line of

text; adapter sockets which cater for PGA (bed of nails) or PLCC

(postage stamp) devices take a little longer, but are straightforward.

[C] The pinouts and pin types (whether input or output etc) for each type

of component (eg LS245) used in the design.  This "library" information

is usually easily entered in minutes by working with the data sheet.

[D] The name (eg IC13), chip type, and location on the board of each

actual component used, together with the names of the nets to

which their pins are to be connected.  This information actually

describes the circuit itself, and can be entered quickly by working

from a rough sketch of a circuit diagram.

Entering coordinate information is made less tedious by allowing the

user to make extensive use of abbreviations.  Defining the shape of a DIL

package is typically a one-liner.

What SDA will produce:

The files required by the BEPI software for further processing (they

are the .CRD, .CPL, .NET, and .PAG files).

A human-readable component list annotated with pin numbers (useful for

the production of hand-drawings), and a net list for checking purposes,

which may also be used for hand wiring if not using the machine.

Preliminaries to the SDA input file format:

Comments:    Anything enclosed in curly brackets is ignored.

Separators:  Blanks (spaces and line-breaks) are used to separate

sytactic items.  Items are names (of chips, nets, etc)

or numbers (coordinates etc) or the special characters

'[',  ']',  '(',  ')',  ',',  ';'.  These special items

also act as separators.

Others:      The special characters '/' and ':' are separators within

items, i.e. they must not have spaces next to them.

They are used to expand abbreviations.

Square brackets:  Used to enclose coordinate information.

Comma:            Used to separate items or groups of items in a list.

Semicolon (';'):  Used to separate groups of item lists, also file sections.

Colon (':'):      Used in abbreviation sequences.

Slash ('/'):      Used to separate items in a sub-list.

Round brackets:   Used to enclose pin lists for packages, chips, components.

Abbreviation conventions:

Sequencing:

A sequence consists of a starting value, an increment, and a final value.

The increment must be a number (and most often this will be 1), the

starting and final values need not be numbers.  The sequence is expanded

by repeatedly adding the increment until the final value is reached.

The examples below illustrate various features.

"1:4"   means "1/2/3/4".            (simple case, with increment omitted)

"7:0"   means "7/6/5/4/3/2/1/0".    (counting down instead of up)

"A1:C2" means "A1/A2/B1/B2/C1/C2".  (counting letters and numbers)

"A1:2C" is meaningless.             (fields do not correspond)

"A2:B1" means "A2/A1/B2/B1".        (numbers counting down, letters up)

"A31:0" means the same as "A31:A0". (leading A of A0 omitted)

"9:2:1" means "9/7/5/3/1".          (increment of 2 specified)

Pairing/Tripling:

Pinouts of a chip, or pin coordinates of a package, are specified as

lists of (signal-name, pin-name) pairs or (pin-name, X-offset, Y-offset)

triples, respectively.  The pairs or triples are formed by simply

writing their constituents next to each other, and separating adjacent

pairs or triples by commas.  For example the pin coordinates of an 8-pin

SIL package might be specified as (1 0 0, 2 0 1, 3 0 2, and so on up to

8 0 7).  This means that pin 1 is at X=0 Y=0, pin 2 at X=0 Y=1, etc.

Note that all the X coordinates are the same, and we abbreviate this

to (1/2/3/4/5/6/7/8  0  0/1/2/3/4/5/6/7), or even further to (1:8 0 0:7).

The distinction between separators ',' and '/' is that the former separates

pairs or triples within a list, while the latter separates simple items

in a sub-list.

The general principle is that a list of triples can be abbreviated to a

triple of lists, and when all the constituent lists are not of the same

length, then the last item in short lists will be replicated until the

list lengths match.

The input file

There are four sections in the file.  The first tells SDA the X-coordinates

of all the columns and the Y-coordinates of all the rows.  The coordinate

unit is the tenth of an inch.

For example, our standard board "750-0433-100", a double-height, extended

depth Eurocard has 26 columns and 87 rows.  We want (column 1, row 1) to

be at X=1, Y=1.  The rows are of course on a regular .1" spacing, and on

this board the columns are on a regular .3" spacing, so the first section

for a file using this board would simply be:

[1:3:76, 1:87]

because the 1:3:76 means 1/4/7/10/.../73/76, i.e. the first X-coordinate

is 1, the second is 4, etc, and the last one is 76; the first Y-coordinate

is 1, the second is 2, etc, up to the last one at 87.

The second section describes the packages, i.e. it gives the relative

coordinates of each pin.  Note that although for things such as DIL

packages pins are usually identified by number, SDA will accept

arbitrary names as pin identifiers.  This section is simply a list of

package descriptions, with the last one followed by a semicolon which

marks the end of the section.  Each package description consists of the

package name, followed in round brackets by a list of triples.  Checks

are made to ensure that no pin is defined twice and that no two pins

share the same pair of coordinates.  See examples above.

The third section describes the chips (including discrete components)

which will be used in the design.  As is the package section, the last

one is followed by a semicolon to mark the end of the section.  Each

chip definition consists of the chip name followed by the package name,

followed in round brackets by four lists of pairs.  Each pair relates a

functional pin name to a physical pin name or number.  The four lists,

separated by semicolons, designate four types of pins (implicit,

input-only, input-output, output-only).  Implicit pins are automatically

connected to a net of the same name whenever the chip is used (useful

for power and ground).  The other pin types are used for checking

purposes (such as ensuring that no net has more than one output-only pin

connected to it).  Checks are made to ensure that no signal or pin is

defined twice, although implicit signals may occur on several pins.

Example of a chip definition:

LS00 DIL14 (.5v 14, .gnd 7; a1:4 1/4/13/10, b1:4 2/5/12/9;; c1:4 3/6/11/8)

Here we define a quad nand gate, with implicit power and ground

connections on pins 14 and 7, as a component with input signals A1 to A4

on pins 1, 4, 13, 10, input signals B1 to B4 on pins 2, 5, 12, 9, output

signals C1 to C4 on pins 3, 6, 11, 8.  The ";;" indicates there are no

input-output signals.

The next section describes the actual circuit design.  It lists a number

of component instances and is followed by a semicolon, which terminates

the section and indeed the input file.

Each component instance description begins with a column and row number,

separated by a comma, enclosed in square brackets.  The row number may,

optionally, be followed by another comma and a rotation parameter.

Rotations may be specified in multiples of 90 degrees (although it is

unlikely that any rotation other than 180 degrees will be appropriate).

The column and row numbers identify the location of the reference point

(usually pin 1) of that chip.  In the early stages of data entry, the

position information (i.e. the square brackets and what goes inside

them) may be omitted if it is only required to check the "correctness"

of the design, but must, of course, be edited in before meaningful files

to drive the wiring machine can be produced.

The position information is followed by the component name (a unique

identifier, e.g. IC3), followed by the chip name, followed in round

brackets by a list of pairs.  The first name in each pair is that of a

pin name as it occurs in the chip definition.  The second name is that

of the net to which the pin is to be connected.  If the net name is the

same as the pin name, it may be omitted.  Checks are made that no pin is

connected to more than one net, and also that every non-implicit pin of

the chip is actually used (these warnings may be suppressed by

"connecting" unused pins to the special pre-defined net NC).

Using SDA (on the APMs)

Prepare your input in a file called <whatever>.SDA, then run the program

ESDL:SDA with <whatever> as parameter.  Progress and error reports are

sent to the terminal and it tells you which files it is writing as it

goes along, these will all be called <whatever>.<something>.

Once this has been done successfully, give the command ESDL:SWRAP <whatever>,

which will sort the net lists and convert them into a form suitable for

sending to the machine.  You should permit the two files <whatever>.CRD

and <whatever>.DAT such that staff can transfer them to the IBM PC which

controls the machine (this is usually done via VAX).

The file ESDL:STARTER.SDA, which is extensively commented, may be used

as a starting point from which to edit in your own design.  In addition,

file ESDL:EXAMPLE.SDA contains a complete example of an SDA design.

RWT, September 1988 revised February 1989.