!{APM Cross-Assembler for Z80/Rainer Thonnes}

EUCSD Zilog Z-80 Assembler  -  User Notes

!<Introduction

This assembler is a very  basic  low-level  program  development  tool.

There  are  no  macros,  and  only  one  data byte or data word or machine

instruction is generated per statement.   Provided that no use is made  of

the  Z-80's  advanced  instructions,  this  assembler  may also be used to

generate code for Intel 8080 and 8085 processors.

The syntax is as nearly as possible identical to that specified in  the

Zilog Z80-CPU technical manual.

Statements  usually  appear  one to a line, but several may appear to a

line  (separated   by   semicolons),   provided   that   (due   to   crude

implementation)  no  more  than  4  bytes of code are generated on any one

line.  N.B. no Z-80 instruction exceeds 4 bytes in length.

A comment, which starts with a '/', may appear  either  on  a  line  by

itself,  or  to  the  right  of any other statement.   Comments occupy the

entire rest of the line, i.e. a semicolon does not terminate a comment.

Names obey the same rules as in  HAL,  i.e.  a  name  is  an  arbitrary

collection  of  letters and digits, starting with a letter, and containing

no other characters, such as spaces.  Although a name may be longer than 6

characters, only the first 6 are used to identify the name uniquely.

Numbers are assumed to be in decimal.   Other  bases  may  be  used  by

prefixing the number by the base and '_', as in IMP-77, e.g. 8_23, 16_3FF.

Constructs  such as 8_16_2A, though silly, are quite legal if you know how

to use  them  (this  example  means  38).   Numbers  may  be  signed  with

'-'(minus)  or '\'(not).   Quoted characters are enclosed in single quotes

(as in IMP), e.g. 'A', and generate 8-bit constants.   The special form of

a  quoted  quote  is  ''' (not '''').   Double quotes are used to generate

16-bit constants, e.g. "AB", which is equivalent to 'A'<<8+'B'.

!>

!<Operands

These fall into the following categories:

!{\tt}

8-bit constant

16-bit constant

8-bit register     <A,B,C,D,E,H,L>

16-bit register    <AF,BC,DE,HL,IX,IY,SP>

index register     <IX,IY>

special register   <I,R>

the top-of-stack   <(SP)>

memory operand     <(constant),(16-bit register),(16-bit register + constant)>

IO-reference       <(C) or (constant)>

!{\rm}

Whether a memory operand or a constant operand refers to an 8-bit or  a

16-bit quantity depends on the context in which it is used.

Assembly-time  expressions,  which  must  evaluate  to a constant or an

operand,  are  worked  out  strictly  left-to-right   with   no   operator

precedence.   Parentheses  may not be used to force any different order of

evaluation.  Operators available are +, -, &, !(or), \(xor), <<, and >>.

Special operands are ".", "$", and "*", all  of  which  mean  the  same

thing,  namely  the  location  counter  at  the  beginning  of the current

instruction.    Use  whichever  one  you  prefer.    E.g.  "DJNZ ."  means

"decrement B until it is zero", a useful delay instruction.

Statement types.   A statement is either an instruction (with two, one,

or no operands), a data statement,  a  name  definition,  or  a  directive

(possibly with parameters).

A  data statement  consists  simply  of  a  constant  (number, name, or

expression).   Normally this constant is dumped as one  byte,  but  if  it

cannot  thus  be  expressed,  it  is dumped as two, least significant byte

first.   Most zero-operand instructions are in fact just constants used as

data  statements.   A  small constant may be forced to occupy two bytes by

prefixing it with the pseudo-instruction "W".

A name definition is either a label  or  an  explicit  definition.   As

expected,  a  label  takes the form "name:", and may occur in front of any

(possibly  null)  statement.    An  explicit  definition  takes  the  form

"name=expression" (which in HAL would be preceeded by "$DEF"), e.g.

BIT4=1<<4; POINTER=HL; ITEM=(POINTER)

!>

!<Directives

These are illustrated here with sample parameters.

!{\tt}

LISTON             turn listing on

LISTOFF            turn listing off

END                Must occur as the last statement in the file

LOC 16_400         Set location counter

W                  Force word data-statement (see above)

BLOCK 13           Reserve  a  block  of  storage  13  bytes  long.   This

directive has a padding side  effect,  and  so  usually

takes the form

BLOCK 16_66-.      which  means  "pad  to location 16_66".   The value 255

(16_FF) is  used  as  padding  data,  unless  specified

otherwise, as in

BLOCK .+8&\7-.,nop which pads with NOPs up to the next 8-byte boundary.

!{\rm}

!>

!<Instructions

These take the form of Zilog-compatible mnemonics, possibly followed by

one or two operands.  They are listed here.

!{\tt}

LD dest,source     'Load'  or  'move'.   Used  both  for  8-bit and 16-bit

quantities.   Note especially: Using (SP) as an operand

has a side-effect, so LD HL,(SP) is the same as POP HL.

Certain operands cannot be moved in one instruction, so

e.g. LD BC,HL must be written as LD B,H; LD C,L.

ADD dest,source    Addition (8 or 16 bit).  Destinations are restricted to

A,  HL,  IX,  and IY.   Similarly ADC and SBC, but with

destinations A and HL only.

INC dest           Both 8-bit and 16-bit.  Similarly DEC.

SUB source         Subtract (destination is A by implication).   Similarly

AND, XOR, OR, CP.

EX op1,op2         The possibilities are:

EX (SP),<HL,IX,IY>  Note: (SP) must come first

EX DE,HL            Note: DE must come first

EX AF,AF            Not EX AF,AF'

EXX                Swop register sets <BC,DE,HL>

PUSH <AF,BC,DE,HL,IX,IY>

POP  <AF,BC,DE,HL,IX,IY>

RST label          Restart (special kind of CALL)

IN reg,dev         Input instruction (dev is either (constant) or (C)).

OUT dev,reg        Output instruction (dev as above).

RLC dest           Rotate left circularly.   Also RRC, RLA, RRA, SLA, SRA,

SRL.    Rotate,   shift,   left,   right,   circularly,

logically, arithmetically.

BIT bit,dest       Bit test.  Also SET, RES.

CALL cond,label    Conditional call

JP cond,label      Conditional absolute jump

JR cond,label      Conditional  relative  jump.    Note  that  the  second

parameter is a label and not a displacement.  Also note

that the 8080/8085 does not have relative jumps.

RET cond           Conditional return

CALL label         Unconditional call

JP label           Unconditional absolute jump

JR label           Unconditional relative jump

RET                Unconditional return.

DJNZ label         Special Decrement B and jump if zero (Z80 only)

!{\rm}

!>

!<Miscellaneous instructions

!{\tt}

LDI,LDD,LDIR,LDDR,      block move

CPI,CPD,CPIR,CPDR,      block compare

INI,IND,INIR,INDR,      block input

OUTI,OUTD,OTIR,OTDR,    block output

RETI,RETN,EI,DI,        interrupt control

IM0,IM1,IM2,            ditto

DAA,                    decimal adjust

CPL,NEG,                complement, negate A

CCF,SCF,                complement, set carry flag

NOP,HALT,DJNZ,          misc

RLCA,RRCA,RLA,RRA       8080-compatible rotates

RLD,RRD                 BCD digit digit rotates

!{\rm}

!>

!<Condition codes for calls, jumps, and returns

!{\tt}

NZ   non-zero

Z    zero

NC   no carry

CY   carry (not C, since C is a register)

PO   parity odd

PE   parity even

P    sign positive

N    sign negative

!{\rm}

!>

!<Error flags

!{\tt}

F  Statement syntactically unacceptable

U  Undefined name

I  Improper operand type

V  Overflow - more than 4 bytes generated on a line

T  Truncation - relative jump label out of range

P  Phase error - usually caused by forward reference in non-W

data statement

*  Name-dictionary full

!{\rm}

!>

!<Deviations from Zilog's syntax spec

1)  The jump qualifier for the condition carry  is  CY,  not  C;  this  is

because the name would otherwise conflict with that of the C-register.

2)  The  destination for relative jumps is specified as a label, not as an

absolute displacement.  So use JR CY,LABEL rather than JR CY,LABEL-$.

Implementations  A)  Legos  _z80  <source>/<object>,<listing>  Object  and

listing  file  names,  if omitted, default to $obj and $list.   Your "LIB"

should be any one of ASSEM, MICROS, or EPROM.  B) Vax $ z80 <file>

The source is taken from <file>.Z80, the object goes  into  <file>.OBJ,

and the listing will be put into <file>.LIS.   Both source and listing are

in normal VMS format, while the object  is  in  IMP  binary  format.   The

symbol Z80 is defined by the public LOGIN file as "@SI1:[MICROS]Z80ASS".

In  both  VAX  and  LEGOS implementations the object file will be 1024n

characters long, suitable for feeding into the EPROM blowing program.

!>

!<Coda

Document dated 11/09/79 revised 15/11/79 RWT.

!>