!<ANALYSE

ANALYSE <cif file>

These commands are used to read a CIF description into  the  internal

data structure in preparation for drawing.   A syntax check is performed

on the CIF as it is read in.  If an error  is  detected  then  the  cell

containing  the  error is marked and only the Bounding Box for this cell

will be drawn if an attempt is made to draw the cell.   There are  three

types of error, namely  WARNINGS (given for the users information):

?  Line 234  Cell FRED contains no geometry

FAULTS (which allow only the bounding box of a cell to be drawn):

*  Line 545  Point only contains one digit

and DISASTERS (the ones which never happen?):

#  Line 999  Internal data structure corrupt - please seek assistance

The  number  typed  is  the  line number of the source text where the

error was detected.   The default extension for the input file is CIF.

!>

!<CALL

CALL  <cell>

This is used to instruct the system to draw the cell specified.   The

cell may be specified by the symbol number which the cell has in the CIF

description of it, or by the comment string if a name has been given  to

the cell. eg.  CALL 32 or CALL PLAPLA.  If cell names are given,  spaces

and  case  is ignored,  but apart from this the names must be identical.

The  call  is done at the current window, level, and layer settings, and

the screen is not wiped before drawing, allowing groups of cells  to  be

built up.

!>

!<CELLS

CELLS <outfile>

This gives a list of the cells defined, their name, number, bounding

box, whether they will be expanded when drawn.  The cell list is placed

in a file if one is specified,  but is displayed on the terminal by

default.

!>

!<DEVICE

DEVICE <device type> <terminal?>

This informs the system of the type of graphics device which is to

be used for drawing.  The device type is given on the same line, eg.

DEVICE CHARLES

DEVICE 963

DEVICE 4014

When the system is entered the null device is set up.  This is may be

superceded by the type of terminal which is connected to the I/O line

if it is a Tektronix, Charles terminal, or HP plotter in the case of VAX.

If a terminal is specified then that terminal is used for any future

graphics output. This is useful for selecting a plotter from another

terminal, after the CIF has been checked.     eg.   DEVICE 7221 TTD7

which sets the system to use the HP plotter connected to line TTD7 for

any further graphics.     SEE ALSO 'HELP USE'

This feature only applies to the VAX implementation of CIFVIEW.

!>

!<DISPOSE

DISPOSE <cell>

This causes all the cells which whose symbol number is greater than or

equal to the specified cell to be deleted.  This is useful if you want to

look a a new CIF file where some of the symbol numbers would clash, without

leaving the CIFVIEW program.  If the parameter is '*' then all the symbols

in the data structure are deleted.

eg.   DISPOSE 11                    { All symbol >= 11 deleted

DISPOSE FRED                  { All symbol >= number of FRED deleted

DISPOSE *                     { All symbols deleted

!>

!<DRAW

DRAW <cell>

This caused the cell specified to be drawn on the current graphics

device.  The cell is drawn so that it will fill the area of the screen

in the case of an interactive terminal, and the screen is cleared before

the drawing is started.

Cells may be specified by the symbol number which the cell has in the CIF

description of it, or by the cells text name.  eg. CALL 32 or CALL PLAPLA.

If cell names are given,  spaces and case are ignored, but apart from this

the names must be identical.

The special from 'DRAW *' can be used to draw the cell defined with the

largest symbol number.

!>

!<END

END or STOP or 'end of input'

These commands are all ways of either :

a) stopping a secondary command file and returning to the console; or

b) stopping the program and returning to command level if there is

no command file in use.

!>

!<GRID

GRID <size> <toggle>

The Grid command will draw a square grid on the graphics terminal

with the size of squares corresponding to <size>.

If <toggle> is omitted, the grid is drawn, and the command has no other

effect.  If <toggle> is set to be ON, then a grid of the size specified

will be drawn after any other cell displayed.    If toggle is set to be

OFF then no grid is drawn, and grids are not drawn until another use of

the GRID command.    Size defaults to 1 if no size is specified.

eg.     GRID 2             { draws a 2 lambda grid for this cell only

GRID 1 ON          { draws a lambda grid after each cell drawn

GRID OFF           { suppresses the drawing of grids

GRID ?             { gives the current grid value

!>

!<HELP

HELP <command>

The following commands are available in the CIFVIEW program.

Further details may be found by giving the name of the command for

which the information is required (eg. HELP CELLS) -

ANALYSE <cif file>                   CELLS   <out file>

DEVICE  <device name>                DISPOSE <cell num>

DRAW    <cell>                       END

GRID    <size> <toggle>              HELP    <command>

IDENT   <layer> <colour> <style>     LAYERS  <layer name>

LEVEL   <number>                     NCIRCLE <number>

ON      <attribute> <cell>           OFF     <attribute> <cell>

PAN     <X Y>?                       POINT   <X Y N?>

REDRAW                               ROTATE  <dir>

SCALE   <number>                     SETLAM  <number>

SIZE    <number>                     STOP

STORE   <file>                       SPEED   <number>

USE     <file>                       VERSION

WCALL   <cell>                       WINDOW  <XL YB XR YT>?

WIPE                                 ZOOM    <XL YB XR YT>?

!>

!<IDENT

IDENT <layer> <colour> <style>

This command instructs CIFVIEW to use the colour specified by <colour>

and the line style specified by <line style> when it is drawing the

layer specified by <layer>.  If colour and layer are omitted then the

default settings are used.   The colour and style may be specifed by their

EDWIN number or by the following names -

colour:  black, blue, red, green

style:   normal, dotted, chain, short-dash, long-dash

eg.

IDENT NG BLUE DOT

IDENT NB 0 S            { Buried contacts in black (0) and short dashes.

IDENT NM                { set to be default (blue & normal) for Metal.

!>

!<LAYERS

LAYERS <layer list>

The layers used in the IC description may be selectively drawn.   The

required layer is selected by this command.   Valid layer names are:

For N-MOS :  ND, NP, NM, NI, NC, NG, NB, NV

For C-MOS :  CD, CP, CM, CW, CC, CG, CA, CN, CV

the first character will be added by CIFSYS if it is omitted.

LAYERS with no parameters causes all layers to be drawn (the default).

Multiple layer names may be specified by the use of + or - between

layer names.

eg.

LAYER CC                    { Draw only CC

LAYERS P + D                { Draw NP + ND or CP + CD

LAYERS NG + I + M           { Draw NP + NI + NM

LAYERS -NB                  { Draw the current layers except for NB

LAYERS +NC                  { Draw the current layers plus NB

!>

!<LEVEL

LEVEL <number>

This  is  a feature to stop large drawings being swamped by excessive

detail.   Once the level has reached the value specified,  only an BB is

drawn for a cell call. The default is level 99. If CIFSYS starts to draw

a cell with more detail than expected,  enter the escape  character  for

the system which you are using  (^C for VAX, ESC on EMAS, ^G on MOUSES).

The response  'Int:'  will be received.   The user can then respond with

a single word, eg 'ENOUGH', followed by return,  in  which  case  CIFSYS

aborts the current plot, and returns to receive the next user command.

If  an  escape  must  be  used  on the HP plotter, the BREAK key must be

depressed first.

!>

!<NCIRCLE

NCIRCLE <number>

If round flashes are  included  in  the  CIF  description,  then  the

circles  are  approximated to polygons for drawing.   The default is for

these polygons to be octagons, but  this  command  allows  the  user  to

increase  the  number of sides used to draw circles.   The default is 8,

and a value of 16 produces quite respectable circles.

!>

!<OFF

OFF <attribute> <cell id>

ON  <attribute> <cell id>

These commands are used to turn attributes of the cell on or off.

Currently there are only two attributes, BB which is whether only a

Bounding Box will be drawn if the cell is instanced, and ID which is

whether cell identification is to appear on any plots.  The settings

of these values are shown by the command CELLS.

If <cell id> is omitted or is '*' then all the cells have their

attribute changed, otherwise only the cell mentioned is altered.

eg.    ON BB Padin             { Set bounding box on cell 'padin'

OFF ID Padout           { Suppress writing 'Padout' when drawing.

ON ID                   { enable all cell names to be drawn.

OFF BB                  { Don't draw BB for any cell.

ON DP Padin             { Draw Ports on cell `Padin'

!>

!<ON

ON  <attribute> <cell id>

OFF <attribute> <cell id>

These commands are used to turn attributes of the cell on or off.

Currently there are only two attributes, BB which is whether only a

Bounding Box will be drawn if the cell is instanced, and ID which is

whether cell identification is to appear on any plots.  The settings

of these values are shown by the command CELLS.

If <cell id> is omitted or is '*' then all the cells have their

attribute changed, otherwise only the cell mentioned is altered.

eg.    ON BB Padin             { Set bounding box on cell 'padin'

OFF ID Padout           { Suppress writing 'Padout' when drawing.

ON ID                   { enable all cell names to be drawn.

OFF BB                  { Don't draw BB for any cell.

OFF DP Padout           { Don't draw ports for cell `Padout'

!>

!<PAN

PAN <X?> <Y?>

This allows the user to specify a new window relative to the current

one, and the last cell which was drawn by the DRAW command is re-drawn

with the revised window setting.

If the line includes the parameters X or Y then the cell is moved by

(X, Y) units, relative to the current window.   The parameter Y may be

omitted, and it defaults to zero.

If no parameter is specified the user can use the cursor to specify two

points whose displacement is used as the distance that the window will

be moved.

eg.     PAN 10                    { Moves the picture 10 units right

PAN 50 50                 { 50 units up and right

PAN -100 -100             { 100 units left and down

PAN                       { Cursor to specify new position

!>

!<POINT

POINT <x> <y> <n?>

The command POINT can be used either to mark a  known  point  on  the

graphics terminal being used,   or  it can use graphics input facilities

of the terminal to enquire the coordinates of a point on the screen.

If a parameter is given a 'marker'  (a small symbol)  is drawn at the

point (X, Y), the type of marker being specified by N. The type defaults

to a cross if it is not specified.  The assignment of numbers to markers

(in the range 0-10) is the same as used by EDWIN.   (See the EDWIN Users

Guide for further details).

If no parameter is specified then the user can use the cursor, mouse,

etc. to identify a point on the screen, and the points coordinates (in

lambda) are reported to the user.

!>

!<REDRAW

REDRAW

This command can be used to re-draw the last cell specifed by the DRAW

command.    It can be used after the user has altered the layers which are

to be drawn, the level which the cell is to be expanded to, or the setting

of the window.   The screen is cleared before the cell is re-drawn.

!>

!<ROTATE

ROTATE <dir>

This command specifies a base rotation which is applied to the commands

DRAW and REDRAW.  Initially there is no base rotation, and the user can

subsequently select either a clockwise or anti-clockwise rotation of 90

degrees.   If the ANALYSE command is used the base rotation is reset to

be zero.

eg.   ROTATE                { Returns to zero base rotation

ROTATE -90            { Rotates the picture 90' CW

ROTATE +90            { Rotates the picture 90' ACW

ROTATE ?              { States what the current base rotation is.

!>

!<SCALE

SCALE <number>

Coordinates in CIF are defined to be 100ths of a micron, as most users

use ILAP to produce their CIF, and work in terms of LAMBDA, the numbers

numbers used by CIFSYS are also in terms of Lambda.  This command may be

used to reset the scale factor which is applied to convert the CIF points

into units of Lambda.   The default is currently a Lambda of 3, thus the

command   SCALE 1  will give the units in terms of 100ths of a micron,

and   SCALE 300  restores the scale to be in terms of Lambda.  Note that

if the CIF was in terms of a Lambda of 2.5 then the command SCALE 250 may

be used.

!>

!<SETLAM

SETLAM <number>

This command may be used to set LAMBDA to a specific dimension on the

HP plotter.  The area that the picture is drawn in is scaled so that

measurements can be made from the plot. If no units are specified

then the number is assumed to be in centimeters,  it is assumed to be

inches if the line ends in '"' or 'ins'.

If the plot can't be drawn on the current paper size, (SIZE), then

the option is given of plotting the picture over multiple sheets.

If a number is not specified, LAMBDA is not significant while plotting,

(the default).

eg.   SETLAM 0.5        { Lambda will be 0.5 cm on further plots.

SETLAM 0.01 "     { Lambda set to 0.01" for future plots.

SETLAM            { Lambda is no longer significant when ploting.

!>

!<SIZE

SIZE <X> <Y?> <OX?> <OY?>

This command may be used to set the area on a plotter, which  is  used

to draw a picture.  If no units are specified then the size is assumed to

be in centimeters.    Inches are used if the line ends with '"' or 'ins'.

A square area is set if the Y size is omitted.  OX and OY are an optional

offset for the lower bounds, which default to zero if they are not given.

If X and Y are both omitted then the user can specify the size by moving

the pen to the paper bounds, and pressing the ENTER button.

eg.  SIZE               { sets plotting area to what user

{ indicates by moving pen.

SIZE 20            { sets plotting area to a 20 cm square.

SIZE 5 4 "         { sets plotting area to a 5" x 4" rect.

SIZE 3.5 6.7 ins   { sets plotting area to 3.5" x 6.7".

SIZE 10 15 5 3     { plotting area is 10 x 15 cm rectangle offset

by 5 cm in the X direction, and 3 cm in Y.

!>

!<SPEED

SPEED <n>

This command may be used to set the plotting speed for HP plotters.

The default (and fastest) is 36 cm/s for the small plotters and 60 cm/s

for the large plotters.  A speed of 10 cm/s is recomended for the drawing

of slides, or if the pens are old.   A number outside the range of speeds

valid for the plotter causes the default to be used.

eg.     SPEED 10

!>

!<STORE

STORE <file name>

If the user wishes to generate an EDWIN Psuedo Display File for

moving to another system, then this command is used. The parameter is

the file name in which CIFVIEW should store the picture description.

After STORE has been called any pictures which are generated by the

use of the DRAW or REDRAW commands are also recorded in the file

which was specified as a parameter to STORE.

Storing can be disabled by specifying the null stream, 'NL:', as the

file name.

!>

!<USE

USE <file name>

This instructs the system to  use  the file specified for further

command input.  Secondary input files may not be nested. If the file

exists  in  the system directory of useful files then this file will

be used,  otherwise the file is searched for in the users directory.

When the file finishes the input is reset to the terminal.

The default extension is COM.

The currently defined 'useful' files are -

USE A4        to set the plotter to draw on A4 paper.

USE A3        to set the plotter to draw on A3 paper.

USE NMOS8     to set the colours so that all 8 plotter pens are used.

USE HPA       to select HP plotter A

USE HPB       to select HP plotter B

USE BIGHP     to select the HP 7580 plotter.

USE SLIDES    to set the paper size and plotter speed for slides.

USE STATE     to give the current 'state' of CIFVIEW.

!>

!<VERSION

VERSION

This command prints the release date of the version of CIFSYS in use

on the users console.   The version should be quoted when reporting any

problems.

!>

!<WCALL

WCALL <cell>

This command can be used to re-draw the cell specifed as the parameter.

It clears the screen before the cell is drawn, but the window is not altered.

This is useful when plotting, where you do not wish to use DRAW and ZOOM.

!>

!<WINDOW

WINDOW <a> <b> <c> <d>

This sets the window to be the box given by the coordinates (a,b) and

(c,d).  Note that this is done explicitly by the commands DRAW and ZOOM.

If no parameters are specified then the device is assumed to have a

method of inputing coordinates graphically, and the user is prompted for

the two window bounds by means of the mouse, cursor etc.

The bounds may be specified in any order.

Windows set by the WINDOW and ZOOM commands are stacked,  (currently

to a depth of 7). This stack is reset by the use of the DRAW command, or

can be unwound by giving the command ZOOM with the parameter '^'.

eg.   WINDOW 50 -50 100 10    { Sets a window to the area specifed

WINDOW ?                { gives the current window value

!>

!<WIPE

WIPE

This causes the 'new frame action' for the device to occur

(eg.  The screen is cleared).

!>

!<ZOOM

ZOOM <a> <b> <c> <d>

This sets the window to be the box given by the coordinates (a,b) and

(c,d), and the last cell specifed to the DRAW command will be re-drawn

with the current window setting.

If no parameters are specified then the device is assumed to have a

method of inputing coordinates graphically, and the user is prompted for

the two window bounds by means of the mouse, cursor etc.

The bounds may be specified in any order.

Windows set by the WINDOW and ZOOM commands are stacked,  (currently

to a depth of 7). This stack is reset by the use of the DRAW command, or

can be unwound by giving the command ZOOM with the parameter '^'.

eg.   ZOOM             { Sets a window with the cursor & redraws cell

ZOOM ^           { returns to the previous window & redraws cell

ZOOM ?           { gives the current window value

!>